Topic D – Medical Physics

4. (a) An isotope of iodine 131I is produced when unstable

tellurium 13152Te undergoes β - decay.

Write a nuclear equation for this decay. (2)

The unstable tellurium is produced when stable 13052Te  is

bombarded with neutrons.

Where might such a reaction be carried out?  (1)

State three factors which must be considered when

choosing a radionuclide for organ imaging.  (3)

131I has a half-life of 8 days and a biological half-life of 21 days.

Calculate the effective half-life of 131I. (2)

On a copy of the axes opposite and above sketch two curves, one

showing the activity of a sample of 131I kept in a laboratory

and the other showing the activity of a sample given to a patient.

Assume both samples have the same initial activity.

Label your curves L (laboratory) and P (patient). (3)

(b) The diagram below shows the main features of an X-ray tube.

Identify features A, B and C. (3)

An X-ray beam has an intensity of 20 MW m-2 at a distance

of 0.10 m from a point source of X-rays.

What will be the intensity 1.0 m from the source?  (2)

(c) Describe how it is possible to obtain information about the

depths of structures in a patient's body by using an ultrasonic A-scan.

You may be awarded a mark for the clarity of your answer. (5)

Explain why a coupling medium is essential between the body surface and the ultrasound probe. (2)

State a suitable substance for use as a coupling medium. (1)

(d) Read the short passage below and answer the questions about it.

The piezoelectric effect is a reversible relationship between mechanical stress and electric potential difference exhibited by certain crystals such as quartz. When a piezoelectric crystal is compressed, positive and negative charges appear on opposite crystal faces producing a potential difference across the crystal. Replacing the pressure by tension reverses this potential difference. If instead an electric potential difference is applied across the crystal then its thickness changes. A piezoelectric crystal placed in an alternating electric circuit will thus expand and contract. These effects are used in ultrasonic generators and detectors. The maximum transfer of energy occurs at a particular frequency called the resonant frequency.  The wavelength of the ultrasound produced by a piezoelectric crystal at its resonant frequency is equal to twice the crystal thickness. [Adapted from Phaidon Concise Encyclopaedia of Science and Technology}

Explain what is meant by a reversible relationship. (2)

Diagram (i) shows the charge distribution on a compressed

piezoelectric crystal. Show the charge distribution on the

crystals on a copy of diagrams (ii) and (iii). (2)

Calculate the thickness of a quartz crystal which will have a

resonant frequency of 1.50 MHz..

( Speed of sound in quartz = 5740 ms-1 )  (3)

What is the benefit of applying the alternating potential

difference at the resonant frequency of the quartz crystal?  (1)

(Total 32 marks)