Topic D – Medical Physics

4. (a) Iodine-123 has a half-life of 13.3 hours. A sample of Iodine-123 has an initial activity of 8000 Bq.

How long will it take for the activity to fall to 1000 Bq?  [2]

On a copy of the axes below sketch a graph to show how the activity of the sample varies with time over

a period of 80 hours.  [3]

An identical sample is administered to a patient. Explain why the measured activity from the patient would not produce the same graph as you have sketched.  [1]

(b) Technetium-99m (99m43Tc) is produced when molybdenum 9942Mo decays.

Write a nuclear equation for this decay.  [1]

The diagram shows a generator for supplying 99m43Tc.

In which part of the generator is the 99m43Tc produced?  [1]

Describe what happens when the tap is opened and air is forced into the space above the saline solution.  [3]

Name a suitable material for the shielding.  [1]

State one advantage of producing 99m43Tc in such a generator.  [1]

9942Mo has a half-life of 67 hours.

Suggest two reasons why this half-life makes 9942Mo appropriate for use in this generator.  [2]

(c) State the function of the following parts of an X-ray tube:

(i) the filament,  (ii) the high voltage supply,  (iii) the glass tube which encloses the anode and filament.  [3]

State two ways in which the radiographer is protected from over-exposure to X-rays.  [2]

On an X-ray photograph, bones show up as bright areas and air spaces such as the lungs produce very dark regions. Explain why. You may be awarded a mark for the clarity of your answer.  [4]

(d) Ultrasound can be used to treat muscle injuries.

Why is a gel applied between the transducer and the patient's skin?  [1]

State one way in which energy is lost from an ultrasound pulse as it passes through a material and state what happens to this energy.  [2]

Beyond the layer of damaged muscle there is bone.

Calculate the reflection coefficient for ultrasound at the muscle-bone interface.  [3]

Specific acoustic impedance of muscle = 1.7 x 10 6 kg m -2 s -1

Specific acoustic impedance of bone = 7.8 x 10 6 kg m -2 s -1

If energy loss in the muscle halves the intensity I of the ultrasound pulse, calculate how much of the intensity of the initial pulse actually enters the bone.  [2]

Total 32 marks