Fuels and Energy

Energy sources:

-release energy at reasonable, controllable but substantial rate

-clean / minimal pollution / not harmful to health

-easily accessible

-cheap / plentiful

| Energy source/process | Advantages | Disadvantages |
| --- | --- | --- |
| coal gasification & liquification | sulfur is removed easier to transport | 30% of energy lost in process |
| coal | readily combustible 40-98% coal | contains impurities (S, H2O, N, P, minerals) |
| crude oil | sulfur easily removed easy to transport little energy required to extract | difficult to assess future reserves high exploration costs |
| gas | no solid residue when burnt cheap to transport few impurities | produces greenhouse gases finite reserves |
| nuclear fusion | limitless fuel (H) waste not very dangerous no need to ship dangerous fuel no risk of plutonium theft | hard to produce plasma hard to obtain enough energy to be viable |
| (direct combustion of) biomass | renewable available fairly non-pollutant | may replace food production (land needed) deplete soil nutrients fermentation produces pollution |
| combustion of waste materials | renewable fairly high octane rating lower CO / NOx emissions can be made from “surplus” plants can grow crops on non-food producing land | uses more energy than returns attracts water into fuel tank (corrosion) lower energy density (ethanol) creates waste ecological problems |
| biogas (thermal & biological conversion) |  |  |
| ethanol |  |  |
| solar energy | clean free, easily harnessed no purification required sustainable no political/economic problems | not economically viable other fuels cheaper must be concentrated not available at night not readily available everywhere |
| passive capture | low cost no moving parts / maintenance long-lasting | must be designed at time of building doesn’t concentrate heat difficult to control backup required |
| active capture | no impact on architecture easy to control & store | more expensive requires maintenance more energy lost through conversion |
| parabolic mirrors |  |  |
| photovoltaic cells | maintenance free, no chemicals easy to use long-lasting | only 13% efficiency (low power output) large collection area required silicon is expensive only work during sunshine |
| wind energy | infinite amount limited cost non-polluting | supply/strength varies (supplement required) run out of places to put farms |
| tidal energy | sustainable | large tides (6-12 m) required |
| geothermal energy |  | hot rocks required |
| dry cell | portable | maximum 1.5 V H2 accumulates on cathode if power drawn too quickly, voltage drops spontaneous reaction (short shelf-life) not rechargeable |
| alkaline battery | longer shelf-life no decline in voltage (1.54V) last longer when in use | 1.54 V max cause corrosion if leak (alkaline) not rechargeable |
| lead-acid storage battery | rechargeable large current (for short time) | large & bulky contains concentrated H2SO4 |
| nickel-cadmium batteries | rechargeable constant 1.4 V light | expensive lower voltage |
| hydrogen-oxygen fuel cell | highly efficient reduced thermal pollution quite light | corrosion if leaks catalyst may fail (Pt/Pd/Rh) |
| pumped storage | no pollution | evaporation occurs large amounts of water required need place to store water lose large amounts of energy in conversion expensive |
| conversion to hydrogen | non polluting light, so easy to capture easy to transport can be used with wind/solar... | gas is bulky, must be pressurised dangerous/flammable energy lost in conversion |
| rechargeable batteries |  | small scale only |