E 1.9

INVESTIGATING HOW THE RESISTANCE OF A PIECE OF WIRE

DEPENDS ON ITS LENGTH

- Look at the title of the investigation and make a prediction. How does the Resistance change when the length changes? You do not need to back up your prediction with theory.

- Set up the following circuit.

- Use a piece of wire approximately 125 cm long and tape it to the bench.

- Connect one end of the wire to a crocodile clip and slide the other crocodile clip along the wire until there is 10 cm of wire in the circuit.

- Set the reading on the voltmeter to 1 Volt by adjusting the variable resistor. Record the current and voltage and hence work out the resistance of 10 cm of wire.

- Now move the variable resistor slightly (say to get 1.5 V on the voltmeter) and measure the current and voltage again. Work out the resistance again, using these new readings.

- Move the variable resistor slightly (say 2 V on the meter) and repeat for a third time. You should now have 3 values for the resistance of 10 cm of wire. Later you will be able to find the average Resistance for 10 cm of wire.

- You now need to repeat the experiment for 20 cm of wire, so slide the crocodile clip along so that there is 20 cm of wire between the crocodile clips.

- Obtain 3 lots of readings for voltage and current (and hence resistance) for 20 cm.

- Repeat again for 30 cm, 40 cm, 50 cm and 60 cm.

- Enter your results in a suitable table

- Plot a suitable graph and draw a conclusion.  Try to explain your results using your knowledge of electricity and atomic structure.

- Write a brief evaluation of the experiment.

- NB IF YOU CARRY OUT THE 2nd EXPERIMENT YOU CAN WRITE 1 EVALUATION FOR BOTH EXPERIMENTS AT THE END.

HINT: If you can’t get V low enough, lower the voltage on the power pack.