E 1.4

RESISTANCE OF A THERMISTOR

| thermistor | 0 -100mA or 0 – 10 mA |
| --- | --- |

Set up the circuit as shown above.

Change the temperature of the thermistor as follows:

- Place it in a beaker with ice & water (low temperature)

- Place it in a beaker with water at room temperature (medium temperature)

- Place it in a beaker with water from a kettle (high temperature)

For each temperature, record the CURRENT in the second column of the table below (if it goes off the scale record it as > 100mA). In the third column say whether you think the RESISTANCE is high, medium or low.

| TEMPERATURE | CURRENT mA | RESISTANCE |
| --- | --- | --- |
| LOW |  |  |
| MEDIUM |  |  |
| HIGH |  |  |

Now connect the THERMISTOR directly to the multimeter. Use the sockets labeled COM and . Set the selector dial to Resistance or  (20K). You are now using a RESISTANCEMETER or OHMMETER.

Ohmmeter

Use the same 3 temperatures as before and record the actual resistances in the table below.

| TEMPERATURE | RESISTANCE  |
| --- | --- |
| LOW |  |
| MEDIUM |  |
| HIGH |  |

SUMMARISE THE BEHAVIOUR OF A THERMISTOR: