# GAUTENG DEPARTMENT OF EDUCATION #### SENIOR CERTIFICATE EXAMINATION BRICKLAYING AND PLASTERING SG TIME: 3 hours MARKS: 300

| REQUIREMENTS: - Answer Book - A2 drawing paper - Calculators may be used. INSTRUCTIONS: - Answer ALL questions. - Use both sides of the drawing paper. - All drawings must be done on the drawing paper in pencil only. - Assume the dimensions of a brick to be: Length: 220mm Width: 110 mm Height: 75 mm |  |
| --- | --- |

| QUESTION 1 - Name FIVE qualities of a good brick. 1.2 State FOUR reasons why some bricks have perforations. 1.3 Name TWO reasons why sand is added to a concrete mix. 1.4 Safety precautions require certain standards to be maintained when working on a building site. Name THREE of these requirements. - State the reasons for using pile foundation. 1.6 Define the term concrete. 1.7 Describe FOUR methods that can be used to cure concrete. | (10) (8) (4) (9) (3) (4) (12) [50] |
| --- | --- |

| QUESTION 2 |  |
| --- | --- |

| - Fully describe how boning rods and sight rails are set up and used in the excavation of a sewer trench which is to have a uniform fall. - Give the standard abbreviations for the following drainage fittings: 2.2.1 Vent pipe - Drain pipe - Soilwater pipe - Wastewater pipe - Gulley 2.3 State the main difference between a vacuum tank and a septic tank. 2.4 Under what circumstances are vacuum tanks installed? | (22) 2x5=(10) (6) (6) [44] |
| --- | --- |

| #### QUESTION 3 |  |
| --- | --- |

| - Describe briefly what the following terms regarding scaffolds mean: 3.1.1 Toe board - Guard rail - Brace - Sole plate - Scaffold boards 3.1.6 Base plate | (4) (4) (4) (4) (4) (4) |
| --- | --- |

| 3.2 You are asked to make checks on a scaffold, to make sure that it is safe for working on. Mention SIX points that you would check. 3.3 Figure 1 illustrates different members of an independent scaffold. Write down the question numbers 3.3.1 - 3.3.12 in your answer book and fill in the missing terms. | (12) (24) [60] |
| --- | --- |

Figure 1

| #### QUESTION 4 |  |
| --- | --- |

| 4.1 To a scale of 1:10, draw the alternate plan courses of a one-and-a-half brick T-junction in Flemish bond. - Describe the method used to mark off a gauge rod. 4.3 A South African type roof truss is constructed out of 38 mm x 114 mm timber and the roof is covered with corrugated sheet iron. The overhang of the roof is 460 mm and rests on a one-brick wall plastered internally. Draw to a scale of 1:10 the elevation of just more than half of the truss and name all the parts. 4.4 Name THREE materials that can be used for coping. | (20) (10) (20) (6) [56] |
| --- | --- |

| #### QUESTION 5 |  |
| --- | --- |

| - Name FIVE places where fire bricks are normally used. - Name FIVE external factors that may cause a chimney to smoke. - Describe the slump test for concrete. - Briefly describe the procedure to plaster a blank wall with one coat of cement plaster. The wall receives a wooden float finish. | (5) (10) (14) (15) [44] |
| --- | --- |

| QUESTION 6 |  |
| --- | --- |

| # Calculate the number of bags of cement required to cast a concrete pile with diameter 600mm and depth 20,5 metres. The concrete mix is 1:2:3. Assuming the volume of one bag of cement to be 0.033 m², calculate the quantity of cement required to the nearest bag. Ignore the bulking effect of sand. 6.2 Calculate the number of tiles of 300 mm x 300 mm required to tile a floor of 12.5 m x 3.45 m. 6.3 Calculate the number of bricks required to build a one-brick wall 20000 mm long and 21000 mm high. Assume that there are 110 bricks per 1m². | (20) (18) (8) [46] |
| --- | --- |

TOTAL:  `300