# GAUTENG DEPARTMENT OF EDUCATION # SENIOR CERTIFICATE EXAMINATION BRICKLAYING AND PLASTERING SG TIME: 3 hours MARKS: 300

# REQUIREMENTS: - Answer Book - A2 drawing paper - Calculators may be used. INSTRUCTIONS: - Answer ALL questions - Use both sides of the drawing paper. - All drawings must be done on the drawing paper in pencil only. - Assume the dimensions of a brick to be: - Length: 220mm - Width: 110mm - Height: 75mm

| # QUESTION 1 - Indicate whether each of the following statements is TRUE or FALSE. Write the numbers 1.1.1 – 1.1.10 with the correct answers alongside each number. |  |
| --- | --- |

| # 1.1.1 Concrete is a mixture of sand, cement and water. |  |
| --- | --- |

| # 1.1.2 Bricks should have a high water absorption capacity. |  |
| --- | --- |

| # 1.1.3 No-fines concrete means concrete without fine aggregate. |  |
| --- | --- |

| # 1.1.4 Admixtures are added to concrete to change the properties of the mix. |  |
| --- | --- |

| # 1.1.5 The centre is the permanent formwork on which arch bricks are supported during construction. |  |
| --- | --- |

| # 1.1.6 A wooden float is used when laying bricks. |  |
| --- | --- |

| # 1.1.7 Reveals are the vertical sides of an opening. |  |
| --- | --- |

| # 1.1.8 Curing means keeping the cast concrete dry. |  |
| --- | --- |

| # 1.1.9 A good trap must retard the flow of water. |  |
| --- | --- |

| # 1.1.10 Fire bricks are bricks that can withstand high temperatures. | 10x1(10) |
| --- | --- |

1.2	Answer the following questions:

| # 1.2.1 What is the main purpose of a flue in a fireplace? | (2) |
| --- | --- |

| # 1.2.2 Define the term cavity wall. | (2) |
| --- | --- |

| # 1.2.3 Moisture can penetrate a wall in one of three ways. Name the THREE ways. | (3) |
| --- | --- |

| # 1.2.4 Name the desired properties of a fire brick. | (6) |
| --- | --- |

| # 1.2.5 Name the FOUR basic causes of accidents on a building site. | (4) |
| --- | --- |

| # 1.2.6 Define the term concrete. | (3) |
| --- | --- |

| # 1.2.7 Name FIVE different types of natural sand. | (5) |
| --- | --- |

| # 1.2.8 Define the term coarse aggregate. | (3) |
| --- | --- |

| # 1.2.9 Define the term fine aggregate. | (3) |
| --- | --- |

| # 1.2.10 Name FOUR advantages of using a cavity wall construction compared with a solid outside wall. | (4) |
| --- | --- |

[45]

| # QUESTION 2 |  |
| --- | --- |

| # 2.1 Briefly explain how Portland cement is manufactured. | (6) |
| --- | --- |

| # 2.2 Name FIVE different types of cement. | (5) |
| --- | --- |

| # 2.3 Briefly describe the correct procedure used to mix concrete by hand. | (11) |
| --- | --- |

| # 2.4 Name and describe FIVE methods that can be used to cure concrete. | (10) |
| --- | --- |

| # 2.5 The slump test is used for determining the desired workability of concrete. Name the equipment required for the test and describe how the test is carried out. | (18) |
| --- | --- |

[50]

| # QUESTION 3 |  |
| --- | --- |

| # 3.1 Draw to an approximate scale of 1:10 the alternate plan courses of a corner formed by two one-and-a half brick walls in English bond. The one wall is 1320mm long with a stopped end and the other wall is 1000mm long. | (20) |
| --- | --- |

| # 3.2 Name FIVE joint profiles that can be used to finish-off unplastered brickwork. | (10) |
| --- | --- |

| # To a scale of 1:10, draw a vertical section through the bottom eight courses of a 270mm cavity wall construction. Show the following detail: - Concrete strip foundation 600mm x 300mm - 75mm concrete floor slab - Screed 20mm - Hardcore - 15 mm internal plaster - Ground level - Damp proof course at floor level height - Wall tie - Weep holes - Cavity filled with concrete up to DPC level | (20) |
| --- | --- |

[50]

| # QUESTION 4 |  |
| --- | --- |

| # 4.1 List FIVE different checks you would carry out to ensure that the tabular scaffolding is safe to work on. | 5x2=(10) |
| --- | --- |

| # 4.2 Name TEN safety precautions which are applicable when erecting or using a scaffold. | 10x2=(20) |
| --- | --- |

| - The different members of the independent tabular scaffold are illustrated in Figure 1. Write down the question numbers 4.3.1 – 4.3.10 in your answer book and fill in the correct missing terms. | 10x2=(20) |
| --- | --- |

[50]

| Figure 1 |  |
| --- | --- |

| # QUESTION 5 |  |
| --- | --- |

| # 5.1 Illustrate by means of neat sketches, the drawing symbols for the following building materials: - Earth filling - Plaster - Hard core - Concrete - Glass | 5x2=(10) |
| --- | --- |

| # 5.2 There are basically four different types of fireplaces. Name the FOUR types. | (8) |
| --- | --- |

| # 5.3 Name FIVE factors that cause chimneys to smoke. | (10) |
| --- | --- |

| # Draw to a scale of 1:10, a vertical section through a fireplace. Your drawing must show the following detail: Concrete strip foundation 600mm x 200mm Back wall 220mm Chimney back 110mm Concrete slab 75mm Back hearth 450mm Screed 30 mm finished with ceramic tiles Front hearth 600mm finished with a 50x60mm trimmer Throat 100 mm Half brick wall bedded on the concrete against the external wall to form the back of the recess which is gathered forward to form a 100mm throat against the lintel. Hearth opening 650mm x 450mm Concrete lintel over opening 220 mm flue Wall above the lintel gathered over (corbeling) to form a 220mm chimney flue. | (27) |
| --- | --- |

[55]

| # QUESTION 6 |  |
| --- | --- |

| # Calculate the number of bricks and pockets of cement, as well as the amount of sand required to build the structure in Figure 2. Use the following: - The walls are 2,40m high. - The doors are 2,00m high and 1,00 m wide. - 110 bricks per 1m² are needed to build a one brick wall. - 55 bricks per 1m² are needed to build a half brick wall. - 1m³ sand per 1000 bricks - 5 pockets of cement per 1000 bricks | [50] |
| --- | --- |

#### Figure 2

TOTAL:  300