GAUTENG DEPARTMENT OF EDUCATION SENIOR CERTIFICATE EXAMINATION ELECTRICIANS WORK SG TIME: 3 hours MARKS: 200

| REQUIREMENTS: - Drawing instruments and an approved calculator. INSTRUCTIONS: - Answer ALL the questions. - Rule off in your answer book after completing each question. - Formulae and calculations, where applicable, must be shown. - A list of formulae, which may be used when applicable, appears on the last page of the question paper. |  |
| --- | --- |

| QUESTION 1 ELECTRICAL CURRENT THEORY |  |
| --- | --- |

| - Determine the inductance of a coil that must be connected in series with a 100 watt, 110 volt incandescent lamp so that it can operate from a 220 volt 50 Hz alternating current supply. (Assume that the resistance of the coil is negligible.) - Calculate the frequency of an alternating voltage which has a cycle time of 10ms. - A parallel circuit consists of a resistor of 60 ohms, a coil with an inductance of 60 mH and a capacitor with a capacitance of 150 microfarad. If the supply is 120V, 50 Hz, calculate: - The current in each component - The total current in the circuit - The phase angle of the circuit - The power factor of the circuit | (12) (2) (10) (3) (2) (2) |
| --- | --- |

| 1.4 An alternating current is represented by the following equation: |  |
| --- | --- |

| i = 60 sin 942,6 t |  |
| --- | --- |

| Calculate: - The effective value of the current - The frequency of the supply - The instantaneous value of the current 0,002 seconds after it is switched on. | (3) (3) (4) |
| --- | --- |

| 1.5 Define the following alternating current terms: - RMS value - Average value | (3) (3) |
| --- | --- |

| 1.6 A series circuit consists of a pure resistance of 20 ohms, and an inductance of 0,07 henry. The circuit is connected to an alternator with a supply voltage of 220 volt. The alternator has two poles and runs at a speed of 3 000 revolutions per minute. |  |
| --- | --- |

| Calculate: - The inductive reactance of the coil - The impedance of the circuit. | (7) (3) |
| --- | --- |

| 1.7 Name THREE types of loads that will cause a lagging power factor. | (3) [60] |
| --- | --- |

| QUESTION 2 INSTRUMENTS AND THREE-PHASE ALTERNATING CURRENT SYSTEMS |  |
| --- | --- |

| - Discuss the vibrating-reed frequency meter under the following headings: 2.1.1 Construction 2.1.2 Operation | (5) (5) |
| --- | --- |

| - Draw a simple circuit diagram to show how a wattmeter should be connected to measure the power in a single-phase circuit. - A wattmeter used to measure the power in an AC circuit reads 2 000 watts, while an ammeter and a voltmeter connected in the same circuit read 10 amperes and 250 volts, respectively. Calculate: | (4) |
| --- | --- |

| 2.3.1 The power factor of the circuit - The active component of the current - The reactive component of the current - The reactive volt-amps. | (4) (3) (3) (3) |
| --- | --- |

| 2.4 Sketch and label the construction of the electron canon of the cathode-ray tube. | (7) |
| --- | --- |

| 2.5 Explain briefly the function of any THREE components in the electron canon of the cathode-ray tube. | (6) [40] |
| --- | --- |

| QUESTION 3 TRANSFORMERS |  |
| --- | --- |

| 3.1 Three single-phase transformers with a turns ratio of 50:1 are connected in delta-star. The supply voltage is 11 kV and the secondary phase-current is 450 A. |  |
| --- | --- |

| - Calculate the secondary line voltage. - Calculate the secondary phase voltage. - Calculate the primary line-current. - Calculate the primary phase-current. - Calculate the transformer rating in kVA. - Draw schematic diagrams of the three single-phase transformers and connect them in delta-star. | (4) (2) (3) (2) (3) (6) |
| --- | --- |

| 3.2 Draw neatly labelled circuit diagrams of the following instrument transformers connected to a supply: - A current transformer - A voltage transformer | (5) (5) [30] |
| --- | --- |

| QUESTION 4 ALTERNATING-CURRENT MOTORS |  |
| --- | --- |

| - Answer the following questions with reference to the three-phase slip-ring motor. 4.1.1 Explain the construction of the rotor. - How is the starting torque improved? - What is the purpose of the short-circuiting and brush-lifting gear? | (4) (3) (2) |
| --- | --- |

| 4.2 Calculate the phase current of a three-phase delta-connected alternator with a line current of 10 ampere. 4.3 What is the purpose of the no-volt coil in a motor starter? - Draw a neatly labelled circuit diagram of a capacitor-start motor. - Draw the control circuit of a direct-on-line starter that will also change the direction of rotation of the motor. - A four-pole, three-phase induction motor is connected to a 380 volt, 50 Hertz supply. The slip of the motor is 5%. | (4) (4) (8) (10) |
| --- | --- |

| Calculate: 4.6.1 The synchronous speed 4.6.2 The rotor speed. | (4) (3) |
| --- | --- |

| 4.7 Briefly explain how the direction of rotation of a three-phase induction motor can be changed. | (2) |
| --- | --- |

| 4.8 A single-phase induction motor has an output power of 2 kilowatts and an efficiency of 88 percent. The power factor is 0,6 and the value of the sinusoidal applied voltage is 240 volt. |  |
| --- | --- |

| Calculate: - The input power - The current flow of the motor. | (3) (3) [50] |
| --- | --- |

| QUESTION 5 ELECTRONICS |  |
| --- | --- |

| - Briefly explain, with the aid of a sketch, a covalent electron bond. - Draw a simple circuit of an amplifier in which an NPN transistor is used. The emitter is common to the input and output circuit. The input and output waveforms must be shown. - State the current carriers in the following materials: 5.3.1 N-type material 5.3.2 P-type material | (5) (6) (2) (2) |
| --- | --- |

| 5.4 What must be kept in mind when any portable electrical appliance is inspected? | (5) [20] |
| --- | --- |

| TOTAL: | 200 |
| --- | --- |

| FORMULA SHEET FORMULEBLAD Z = R2+(XL≈Xc)2VR=IT x RIT=VTZ Z = R2+XL2 Z = R2+Xc2 VL=ITxXL Vc=ITxXc IT = IR2+(Ic≈IL)2IR=VRR;IL=VLXL; IC=VcXc; Cosθ=IRIT XL =2πfL Xc = 12πfC P = V x I x Cos θ Cos θ = RZ Tan θ = XL−XcR ; Cos θ = PVA P = I2R Iact = I x cosθ Ireact = I x sinθ Star/ster delta IL = Iph IL = 3x Iph VL = 3xVph VL = Vph F = pN60S=Ns−NRNSx100%NR=fp(1−s) P = 3xVLxILxcosθ S = 3 x VL xIL;VpVs=NPNs=IsIporofV1V2=N1N2=I2I1 Efficiency = OutputInput Rendement = AfvoerInvoer |  |
| --- | --- |