GAUTENG DEPARTMENT OF EDUCATION SENIOR CERTIFICATE EXAMINATION MOTOR BODY REPAIRING SG TIME: 3 hours MARKS: 200

| REQUIREMENTS: - An approved (non-programmable scientific) calculator. Candidates must provide their own calculators. - Drawing instruments INSTRUCTIONS: - Answer ALL the questions in the answer book. |  |
| --- | --- |

| QUESTION 1 |  |
| --- | --- |

| 1.1 What aspects should be considered before replacing a body section? | (8) |
| --- | --- |

| 1.2 Describe FOUR different methods of hammering employed in body repairing. | (4) |
| --- | --- |

| 1.3 Name FIVE safety regulations to be followed when working with compressed gas cylinders. | (5) |
| --- | --- |

| 1.4 Name TWO advantages of spot-welding. | (2) |
| --- | --- |

| 1.5 Name TWO methods of measuring according to which the alignment of a door opening can be checked. | (2) |
| --- | --- |

| 1.6 What is the purpose of the catalyst when working with fibre-glass? | (2) |
| --- | --- |

| 1.7 Name TWO advantages of fibre-glass over steel. | (2) |
| --- | --- |

| 1.8 Briefly describe the cutting technique of a mild-steel plate with the oxy-acetylene cutter. | (10) |
| --- | --- |

| 1.9 What is the difference between the integral-type chassis and the conventional-type chassis? | (2) |
| --- | --- |

| 1.10 Why is a rubber block used when sanding body-putty? | (3) [40] |
| --- | --- |

| QUESTION 2 |  |
| --- | --- |

| 2.1 List all the steps that should be taken into consideration before wheel alignment is checked. | (5) |
| --- | --- |

| 2.2 Describe toe-out on turns by means of a sketch. | (10) |
| --- | --- |

| 2.3 Describe negative camber by means of a sketch. | (10) |
| --- | --- |

| 2.4 Name TWO causes of the thread of front tyres wearing down quickly. | (2) |
| --- | --- |

| 2.5 Why is toe-in important? | (3) [30] |
| --- | --- |

| QUESTION 3 |  |
| --- | --- |

| 3.1 Describe step-by-step the procedure which is carried out when a hole in a fibre-glass panel is repaired. | (10) |
| --- | --- |

| 3.2 List FIVE tools used when working with fibre-glass. | (5) |
| --- | --- |

| 3.3 List TEN tools that are used in body repairing. | (10) [25] |
| --- | --- |

| QUESTION 4 |  |
| --- | --- |

| 4.1 A vehicle was involved in an accident and was damaged on the right-hand side. The door must be replaced, but the door pillar was pushed slightly backwards. This vehicle has an integral-type chassis. Describe in detail how repairs should be carried out and a new door fitted. | (18) |
| --- | --- |

| 4.2 What is meant by form changing in steel? | (5) |
| --- | --- |

| 4.3 What is meant by elasticity of steel? | (2) [25] |
| --- | --- |

| QUESTION 5 |  |
| --- | --- |

| 5.1 Describe, step-by-step, how a crack in a conventional-type chassis will be welded and reinforced. | (20) |
| --- | --- |

| 5.2 5.2.1 Name TWO advantages of a conventional-type chassis. | (2) |
| --- | --- |

| 5.2.2 Name TWO advantages of an integral-type chassis. | (2) |
| --- | --- |

| 5.3 Which of the following two operations is carried out first in the case of an integral-type chassis? |  |
| --- | --- |

| 5.3.1 Straightening the body |  |
| --- | --- |

| 5.3.2 Straightening the frame |  |
| --- | --- |

| 5.3.3 Both | (2) |
| --- | --- |

| 5.4 What will the consequences be when a motor car with an out-of-alignment chassis frame is used on the road? | (4) [30] |
| --- | --- |

| QUESTION 6 |  |
| --- | --- |

| 6.1 Name FIVE safety precautions to be followed in the spray booth. | (5) |
| --- | --- |

| 6.2 State the reasons for an orange-peel effect on a painted surface. | (2) |
| --- | --- |

| 6.3 Why does paint “flow” during the application of spray paint. | (2) |
| --- | --- |

| 6.4 Briefly explain the operation of the spray-gun. | (5) |
| --- | --- |

| 6.5 Briefly describe how body-putty is applied to a surface. | (5) |
| --- | --- |

| 6.6 What should the correct distance be between the spray gun and the painted surface? | (2) |
| --- | --- |

| 6.7 Briefly describe how heat shrinking is done. | (5) |
| --- | --- |

| 6.8 Name TWO important aspects to be considered when using a compressor for spray-painting. | (4) [30] |
| --- | --- |

| QUESTION 7 |  |
| --- | --- |

| 7.1 Explain how a panel should be prepared for spray-painting. | (10) |
| --- | --- |

| 7.2 Why is it not advisable to leave thinners in the spray gun after it was cleaned? | (4) |
| --- | --- |

| 7.3 Name THREE characteristics of a good pigment. | (3) |
| --- | --- |

| 7.4 Name THREE groups of pigments. | (3) [20] |
| --- | --- |

| TOTAL: | 200 |
| --- | --- |