| GAUTENG DEPARTMENT OF EDUCATION SENIOR CERTIFICATE EXAMINATION MOTOR MECHANICS SG TIME: 3 hours MARKS: 200 |  |
| --- | --- |
| REQUIREMENTS: - Calculator and drawing instruments INSTRUCTIONS: - This examination paper consists of SECTION A and SECTION B. - Section A is COMPULSORY. - Answer any FIVE questions from Section B. - Answer QUESTION 1 (1.1 – 1.15: Multiple-choice Questions) on the answer sheet on the inside cover of your answer book. All the other questions should be answered in your answer book. - A formulae sheet is included on page 9. - Sketches should be large, neat and well-proportioned. Please label all parts. |  |

| SECTION A |  |
| --- | --- |

| QUESTION 1 MULTIPLE-CHOICE QUESTIONS |  |
| --- | --- |

| 1.1 It is advisable to keep the ________ when lifting heavy objects. |  |
| --- | --- |

| - legs apart and use the arms - back straight and use the legs - legs straight and use the back | (2) |
| --- | --- |

| 1.2 The international colour code for a 220 Volt, three-core cable is |  |
| --- | --- |

| - earth red, live blue, neutral green and yellow. - earth green, live red, neutral black. - earth green and yellow, live brown, neutral blue. | (2) |
| --- | --- |

| 1.3 Fuel should be stored in one of the following places: |  |
| --- | --- |

| - The store - A fireproof room - The mechanical workshop | (2) |
| --- | --- |

| 1.4 When connecting a battery, the first step would be to connect the |  |
| --- | --- |

| - live conductor. - negative conductor. - earth conductor. | (2) |
| --- | --- |

| 1.5 Petrol used in internal combustion engines consists of hydrogen and |  |
| --- | --- |

| - nitrogen. - carbon. - oxygen. | (2) |
| --- | --- |

| 1.6 Which one of the following refers to the calibrating of the injector pump? |  |
| --- | --- |

| - The beginning of injection - The end of injection - The quantity of fuel injected | (2) |
| --- | --- |

| 1.7 What would a foam-type fire extinguisher be used for generally? |  |
| --- | --- |

| - Solid matter - Electrical fires - Flammable liquids | (2) |
| --- | --- |

| 1.8 Never work under a vehicle when the engine is running because |  |
| --- | --- |

| - the engine might overheat. - carbon monoxide is a heavy gas. - it is not very comfortable. | (2) |
| --- | --- |

| 1.9 The __________ is not classified as a positive displacement blower. |  |
| --- | --- |

| - centrifugal-type blower - vane-type blower - roots-type blower | (2) |
| --- | --- |

| 1.10 The firing order for a six-cylinder in-line engine is: |  |
| --- | --- |

| - 1, 4, 2, 5, 3, 6 - 1, 4, 3, 6, 2, 5 - 1, 5, 3, 6, 2, 4 | (2) |
| --- | --- |

| 1.11 Torsional vibrations by the crankshaft could be caused by |  |
| --- | --- |

| - static unbalance. - dynamic unbalance. - effect of the torque resulting from power strokes. | (2) |
| --- | --- |

| 1.12 The diode in an electrical circuit allows current to |  |
| --- | --- |

| - flow in both directions. - flow in one direction. - stop flowing in the circuit. | (2) |
| --- | --- |

| 1.13 The voltmeter is connected in |  |
| --- | --- |

| - series. - parallel. - None of the above. | (2) |
| --- | --- |

| 1.14 The two-speed final drive is engaged in high speed when the engaging gear is |  |
| --- | --- |

| - locked. - free. | (2) |
| --- | --- |

| 1.15 In the compression ignition injection, pump injection ceases when the helix of the pump plunger |  |
| --- | --- |

| - uncovers the spill port. - closes the spill port. - closes the inlet port. | (2) 2x15=[30] |
| --- | --- |

| QUESTION 2 |  |
| --- | --- |

| 2.1 What effect would a leaking cylinder head gasket have on an internal combustion engine? | (2) |
| --- | --- |

| 2.2 Why does a bi-metal strip deflect when heated? | (4) |
| --- | --- |

| 2.3 Name the THREE connecting terminals of the transistor. | (6) |
| --- | --- |

| 2.4 A motorcar engine develops a torque of 560 Newton metres at 3 000 revolutions per minute. Calculate the brake power of the engine. | (4) |
| --- | --- |

| 2.5 Define effective pump stroke. | (4) |
| --- | --- |

| 2.6 What is the purpose of the steering box in the motor vehicle? | (2) |
| --- | --- |

| 2.7 Define the catalytic cracking process. | (6) |
| --- | --- |

| 2.8 State TWO alternative firing orders for four-cylinder engines. | (4) |
| --- | --- |

| 2.9 State TWO advantages of epicyclic gear trains. | (4) |
| --- | --- |

| 2.10 The sketch in Figure 1 shows a linkage-type power steering with the front wheels in a straight-ahead position. Draw this sketch in your answer book showing the position of the various components when the front wheels are turned to the right. Label fully. | (20) |
| --- | --- |

| 1 2 11 12 13 7 10 3 4 10 5 6 7 9 8 |  |
| --- | --- |

| Figure 1 |  |
| --- | --- |

| 2.11 State TWO advantages of a power-steering steering box in comparison to the conventional steering box. | (4) |
| --- | --- |

| 2.12 Read the following statements carefully and indicate whether they are TRUE or FALSE by writing down only TRUE or FALSE in your answer book next to the relevant question number. |  |
| --- | --- |

| - The electric motor used for windscreen wipers is known as an alternating motor. - The indicated power could be used as a measure to determine the theoretical power developed by an engine. - In the torque converter, torque increase could be provided automatically. - The workshop is a place to work hard and play carefully. - Fuel filters should remove any foreign solid matter from the fuel. - Transistors are not sensitive to engine vibrations. | (1) (1) (1) (1) (1) (1) |
| --- | --- |

| - Commencement of injection varies in the petrol injection system with the inverted helix. - The number of gear ratios provided by the gearbox is doubled by the use of a two-speed final drive. - Power unbalance is caused by the uneven spreading of the mass around the axis of rotation of the crankshaft. - The power balance of a V-4 engine is good. | (1) (1) (1) (1) [70] |
| --- | --- |

| TOTAL FOR SECTION A: | [100] |
| --- | --- |

| SECTION B |  |
| --- | --- |

| Answer any FIVE questions from this section. |  |
| --- | --- |

| QUESTION 3 |  |
| --- | --- |

| 3.1 Draw a diagrammatic layout of a transistor ignition system equipped with contact points. | (10) |
| --- | --- |

| 3.2 Draw a neat sketch of a transistor symbol. | (4) |
| --- | --- |

| 3.3 State THREE advantages of the transistor ignition system in comparison with the conventional system. | (6) [20] |
| --- | --- |

| QUESTION 4 |  |
| --- | --- |

| 4.1 By means of simple sketches illustrate the following fuel injection pump components: |  |
| --- | --- |

| 4.1.1 Delivery valve 4.1.2 Pump element showing the plunger on maximum fuel delivery | (8) (4) |
| --- | --- |

| 4.2 What component should be removed from the fuel injection pump to be able to set spill timing? | (2) |
| --- | --- |

| 4.3 Name TWO types of compression ignition fuel injection pumps. | (4) |
| --- | --- |

| 4.4 At what stage during the effective pump stroke does ignition take place in a C.I. engine? | (2) [20] |
| --- | --- |

| QUESTION 5 |  |
| --- | --- |

| 5.1 The following data refers to a two-cylinder two-stroke engine which was tested with a Prony brake: |  |
| --- | --- |

| Mean effective pressure : 900 kPa Length of stroke : 86 mm Cylinder bore : 84 mm Revolutions per minute : 2 000 Brake arm length : 400 mm Spring balance reading : 25 kg |  |
| --- | --- |

| Assume 1 kg = 9,8 N |  |
| --- | --- |

| Calculate: |  |
| --- | --- |

| - Indicator power - Brake power - Mechanical efficiency | (6) (4) (2) |
| --- | --- |

| 5.2 Draw a typical cylinder pressure diagram for a four-stroke petrol engine having a compression ratio of 9:1. | (8) [20] |
| --- | --- |

| QUESTION 6 |  |
| --- | --- |

| 6.1 Illustrate the following wheel alignment angles by means of simple sketches: |  |
| --- | --- |

| - Positive camber - Negative castor - King pin inclination | (4) (4) (4) |
| --- | --- |

| 6.2 Name TWO factors that could affect toe out on turns. | (4) |
| --- | --- |

| 6.3 Name TWO advantages of negative castor. | (4) [20] |
| --- | --- |

| QUESTION 7 |  |
| --- | --- |

| 7.1 Draw a neat sectional sketch of a torque converter. | (12) |
| --- | --- |

| 7.2 State the TWO basic differences in construction between a fluid coupling and a torque converter. | (4) |
| --- | --- |

| 7.3 State TWO advantages of a torque converter. | (4) [20] |
| --- | --- |

| QUESTION 8 |  |
| --- | --- |

| Study the sketch of the multibarrel carburettor below and answer the questions that follow. |  |
| --- | --- |

| 3 1 10 A B 9 9 C 2 5 7 6 4 8 |  |
| --- | --- |
| Figure 2 |  |

| 8.1 Name the components marked 1–10. | (10) |
| --- | --- |

| 8.2 Name the carburettor circuits indicated by A, B and C. | (6) |
| --- | --- |

| 8.3 Name TWO types of constant vacuum carburettors. | (2) |
| --- | --- |

| 8.4 What component is responsible for the jet size in the constant vacuum carburettor? | (2) [20] |
| --- | --- |

| TOTAL FOR SECTION B: | [100] |
| --- | --- |

| TOTAL: | 200 |
| --- | --- |

| FORMULAE SHEET |  |
| --- | --- |

| - F = m x a |  |
| --- | --- |

| - Work = F x distance |  |
| --- | --- |

| - T = F x R |  |
| --- | --- |

| - Power = F x distancetime |  |
| --- | --- |

| - Power = M.E.P. x π x D2 x stroke length x r/s x number of cylinders4 x 2 |  |
| --- | --- |

| - Power = M.E.P x π x D2 x stroke length x r/s x number of cylinders4 |  |
| --- | --- |

| - IP = PLANn |  |
| --- | --- |

| - Brake power = F x 2 π R x N |  |
| --- | --- |

| - Brake power = 2 π NT |  |
| --- | --- |

| Mechanical efficiency = B.P.I.P. x 1001 |  |
| --- | --- |

| C.R. = SV + CVCV |  |
| --- | --- |

| Area = π D24 |  |
| --- | --- |

| Stroke volume = π D2L4 |  |
| --- | --- |