| # GAUTENG DEPARTMENT OF EDUCATION SENIOR CERTIFICATE EXAMINATION TECHNIKA (ELECTRICAL) HG TIME: 3 hours MARKS: 300 |  |
| --- | --- |

| REQUIREMENTS: - Drawing instruments and an approved calculator. |  |
| --- | --- |

| INSTRUCTIONS: - Answer ALL the questions. - Sketches and diagrams must be clear and writing must be legible. - Formulae and calculations must, where applicable, be indicated. - A list of formulae, which may be used when applicable, is provided on the last page of the question paper. |  |
| --- | --- |

| # QUESTION 1 # OCCUPATIONAL SAFETY - Briefly describe the safety precautions which must be complied with before a portable electric appliance is taken into use. 1.2 State TWO important causes of industrial accidents. | (5) (2) |
| --- | --- |

| 1.3 In case of an accident taking place in the workshop, i.e. someone has cut himself/herself and there is bleeding, what precaution would you take to ensure that you would not be infected with the HIV virus? | (1) [8] |
| --- | --- |

| QUESTION 2 ELECTRICAL CURRENT THEORY - A resistance of 10 ohms, an inductance of 200 milli-henrys and a capacitor of 100 micro-farads are connected in series across a 250V 50 Hz supply. Calculate: 2.1.1 The circuit impedance - The circuit power factor - The reactive component of the current 2.2 A coil with a resistance of 10 ohms and an inductance of 50 milli-henrys is connected in parallel with a capacitor of 200 micro-farads across a supply voltage represented by the equation. V = 100 sin (628+ 85xΠ180° ) volt. Calculate: - The supply frequency - The resonate frequency of the circuit - The dynamic impedance of the circuit - The value of the circulating current | (9) (3) (7) (3) (3) (3) (3) |
| --- | --- |

| # 2.3 In a certain balanced, star connected three-phase circuit with an inductive load, the line to line voltmeter reading is 380 V, the ammeter reading 10 A and the wattmeter reading 3 kW. Determine: - The phase voltage - The phase current - The power factor of the load - The phase impedance - The phase resistance | (3) (2) (3) (3) (3) [45] |
| --- | --- |

| # QUESTION 3 SINGLE- AND THREE-PHASE ALTERNATING CURRENT SYSTEMS - State THREE characteristics of a balanced, star connected system. - A resistance load draws 15A when connected to a 125 V supply. An inductor is connected in series with this load to a 240 V, 50 Hz supply. The current through the load remains the same. Ignore the resistance of the load and 3.2.1 calculate the inductance of the coil in mH. - calculate the phase difference between the current and the applied voltage and state whether it is leading or lagging. - calculate the voltage across each component. - draw a neat labelled voltage phasor diagram of the circuit in good proportion. - draw a neat labelled impedance triangle of the circuit to any applicable scale. | (3) (14) (5) (6) (6) (4) [38] |
| --- | --- |

| QUESTION 4 TRANSFORMERS - Three single phase transformers are connected in delta to a 3 kV supply. The secondaries of the transformers are connected in star and supply 415 V line voltage to a balanced load. The power in the load is 150 kW at a power factor of 0,8 lagging. The primary is wound at 4 V per turn. Assume 100% efficiency. 4.1.1 Draw a neat labelled diagram to show the connection of the transformers. - Calculate the number of primary windings. - Calculate the number of secondary windings. - Calculate all the current values in the circuit. 4.2 Describe fully why the secondary windings of a current transformer may never be left open-circuited. 4.3 State TWO disadvantages and TWO advantages of an autotransformer. | (5) (5) (5) (13) (6) (4) [38] |
| --- | --- |

| QUESTION 5 ALTERNATING CURRENT MOTORS - Describe how the direction of rotation of a capacitor run induction motor can be changed. - Briefly describe TWO methods that are used in starters to limit the starting current of three phase squirrel cage motors. - A four pole squirrel cage induction motor is connected to a 380 V alternating current supply with a periodic time of 0.02 seconds. The motor slip is calculated to be 0,04. Calculate: 5.3.1 The supply frequency - The rotor speed 5.4 Sketch a neat labelled diagram to show the internal wiring of a capacitor start motor. 5.5 Give the phase order of a three phase system. | (2) (6) (3) (6) (7) (3) [27] |
| --- | --- |

| QUESTION 6 SEMI-CONDUCTORS |  |
| --- | --- |

| 6.1 With reference to Figure 6.1 answer the questions that follow. |  |
| --- | --- |

| +50 V VG = –3 V C1 C2 R1 T1 –2V –3V –4V |  |
| --- | --- |
| # Figure 6.1 |  |

| - Identify the above circuit. - Identify T1. - Briefly describe the function of R1. - What is the phase relation between the input and the output signals? - Draw the simbol of T1 in your answer book and identify each terminal. - Discuss TWO advantages of the above circuit as compared to a functionally equivalent bipolar transistor circuit. | (2) (2) (2) (1) (3) (4) [14] |
| --- | --- |

| QUESTION 7 AMPLIFIERS - Calculate the turns ratio of a transformer which matches an 8 ohms speaker to an amplifier with an output impedance of 400 ohms so as to obtain maximum power transfer. - The input power of an amplifier is 20 W. Calculate the gain, in decibels, if it supplies 50 W to a load. - Sketch a typical frequency characteristic curve of a mutually coupled amplifier. - Draw a diagram of a Darlington amplifier with NPN transistors. - A simple common emitter amplifier is connected to a load resistor of 4k-ohms. The voltage supply to the circuit is 10V. 7.5.1 Calculate the co-ordinates of the loadline. 7.5.2 Draw the load line according to an appropriate scale. | (4) (3) (4) (7) (6) (5) [29] |
| --- | --- |

| QUESTION 8 SWITCHING AND CONTROL CIRCUITS - Fully describe why it is sometimes necessary to connect a regulator to the output of a dc voltage source so as to form a regulated voltage supply. - Describe, with the aid of a circuit diagram and a brief discussion, how full wave lamp dimming can be obtained in an ac circuit. - Sketch a circuit diagram of a circuit capable of limiting a sinus oidal voltage with a maximum value of 10V, to 5V in both directions. All input and output waveforms must be shown. | (5) (12) (8) [25] |
| --- | --- |

| # QUESTION 9 OSCILLATORS Draw a neat labelled diagram of a crystal controlled Harley oscillator and name the most important reason for including the crystal in the circuit. | (12) [12] |
| --- | --- |

| QUESTION 10 APPLICATION OF OPERATIONAL AMPLIFIERS Draw a diagram of a bi-stable multivibrator which uses an operational amplifier. | (5) [5] |
| --- | --- |

| QUESTION 11 – COMPUTER PRINCIPLES Sketch a logic combination circuit for the following Boolean equation: X = (A + B) (Ā + B) - Explain the meaning of the following terms briefly: 11.2.1 Combination circuit - Clock pulse - Full adder 11.3 Draw a schematic diagram of a clocked RS latch consisting of nand gates. Use a truth table to show the output of the latch for all possible input combinations. Assume in each case that the clock pulse = 1. 11.4 An electronics engineer is approached to design a circuit consisting of three switches, A, B and C, which will allow the current to flow under the following conditions: * Switches A and C open and B closed * Switches B and C open and A closed * Switches A and C closed and B open * All three switches closed - Draw up a truth table for the circuit. - Derive a logical expression for the circuit. - Simplify the Boolean expression and sketch a logic circuit of the expression. | (4) (2) (2) (2) (12) (8) (4) (10) [44] |
| --- | --- |

| QUESTION 12 # MEASURING INSTRUMENTS - Draw a neat labelled, block diagrammatic representation of an induction meter. 12.2 The block diagram below represents a digital capacitance meter. | (9) (6) |
| --- | --- |

| A B C R E D F ###### GATE CONTROL COMPARATOR OSCILLATOR CONVERTER EARTH |  |
| --- | --- |

| Identify the components A – F. | [15] |
| --- | --- |

| TOTAL: | 300 |
| --- | --- |

# FORMULES / FORMULAE

x6 = 2πfL								IE = I8 + Ic

Tydkonstante / Time constant  t = RC			fr = 12πLC

Z = R2÷XL−Xc2						Cos Φ = PP skynbaar / apparent

VR = IxR

VL = IxXL

Ir = I sin Φ : Ia = I cos Φ					R = X cos Φ	VC = IxXC

## Ster / Star

P = VL3 : IP = IL							Z = VI		P 3 IL EL cos Φ

###### Delta

VP = VL : IP = IL3						Z = LCR

I = VZ							f = 1L 		VpVs = NpNs = IsIp

ƒ = W2π							Ic = VceRL

Xc = 12πfC						Rendement / Efficiency = Uitset/OutputInset/Input

Cos Φ = RZ							Ic = V αCL

S =Ns−NrNs							AF = Av / 1÷BAv

β=IcIB								N = 10Log P2P1

Q = XLR									NpNs = ZPZS

Ns = fp	Nr = Ns - S)