| GAUTENG DEPARTMENT OF EDUCATION SENIOR CERTIFICATE EXAMINATION TECHNIKA (ELECTRICAL) HG TIME: 3 hours MARKS: 300 |  |
| --- | --- |
| REQUIREMENTS: - Drawing instruments and an approved calculator INSTRUCTIONS: - Answer ALL the questions. - Sketches and diagrams must be clear and writing must be legible. - Formulae and calculations must, where applicable, be indicated. - A list of formulae which may be used where applicable, is provided on page 11 of the question paper. You may detach it for easy reference. |  |

| QUESTION 1 OCCUPATIONAL SAFETY - Explain how you would apply artificial respiration to a person who has stopped breathing as a result of an electric shock. | (4) |
| --- | --- |

| 1.2 Explain how Aids can be spread from one person to another. | (3) [7] |
| --- | --- |

| QUESTION 2 ELECTRICAL CURRENT THEORY - A coil with an insignificantly small resistance has an inductance of 75 mH and is connected in series with a 0,22 micro-Farad capacitor and a 20 Ohms resistor. The supply voltage is 110 Volts, 500 Hertz. Calculate accordingly: 2.1.1 The maximum current flow in the circuit - The voltage over the reactive parts of the circuit when maximum current flow takes place - The Q-factor of the circuit - The phase angle when maximum current flow takes place in the circuit | (4) (7) (3) (4) |
| --- | --- |

| 2.2 A parallel resonance circuit consists of a capacitor of 100 micro-Farads and a coil of which the inductance is 25 mH and the resistance 5 Ohm. The supply voltage is 25 Volts. - Draw a neat labelled diagram of the circuit. - Calculate the dynamic impedance of the circuit. - Calculate the value of the supply current. - Calculate the value of the circulating current. - At which frequency will the circuit resonate? - Draw a neat labelled current phasor diagram (not according to scale, but in good relation to the circuit). | (4) (3) (3) (3) (3) (3) |
| --- | --- |

| 2.3 Explain the following terms: - The ampère - Frequency - Phasor - Electromagnetic induction | (4) (2) (2) (4) |
| --- | --- |

| 2.4 Describe the meaning of each of the following concepts: - Q factor - Active component of alternating current - Efficiency | (2) (2) (3) [56] |
| --- | --- |

| QUESTION 3 SINGLE- AND THREE-PHASE ALTERNATING CURRENT SYSTEMS - Three identical coils are connected in star to a 400 volts, 50 hertz three-phase supply. The total power used by the coils is 1,5 kW at a power factor of 0,2. Determine the resistance and the inductance of each coil. | (23) |
| --- | --- |

| - Prove that the total power used in a star connected three-phase system is: P = 3.VL.IL.Cos ø watts. | (7) [30] |
| --- | --- |

| QUESTION 4 TRANSFORMERS - The winding of an auto-transformer consists of 1 000 turns, and is used to transform 250 volts to 25 volts. 4.1.1 State the disadvantage of this specific application in which the voltage is stepped down to 25 volts. - Make a neat labelled diagrammatic representation of the transformer. - Calculate the position of the tap on the winding where the 25 volts are obtained (number of secondary turns). | (3) (4) (4) |
| --- | --- |

| 4.2 Three identical single phase transformers, each having a turn ratio of 50:1, are connected in star-delta across a 33 kV supply. The load connected to the transformers is a three-phase star connected motor with a power factor of 0,8 and an efficiency of 90%. - Draw a neat labelled circuit diagram of the transformers, the motor and all the inter-connections. - Calculate the voltage across each motor winding. - Calculate the current through each motor winding if the output power of the motor is 20 kW. | (8) (11) (9) [39] |
| --- | --- |

| QUESTION 5 ALTERNATING CURRENT MOTORS - Explain the term synchronous speed of an electric motor. | (2) |
| --- | --- |

| 5.2 A four-pole three-phase induction motor is connected to a 380V/50Hz supply. Calculate the synchronous speed. | (4) |
| --- | --- |

| 5.3 Sketch a neat labelled diagram to show the internal wiring of a capacitor start motor. | (7) |
| --- | --- |

| 5.4 What is the most important difference between a starting capacitor for an electric motor and a regular electrolytic capacitor? | (2) |
| --- | --- |

| 5.5 How can the direction of rotation of a single-phase capacitor run motor be altered? | (2) |
| --- | --- |

| - A four-pole three-phase induction motor is connected to a 380 V, 50 Hz-supply. Calculate: 5.6.1 The synchronous speed 5.6.2 The rotor speed if the slip is 4% | (3) (3) |
| --- | --- |

| - A single-phase alternating current motor is connected to a 500V, 50 Hz supply. The motor develops 20 kW at a lagging power factor of 0,8. If the efficiency of the motor is 84%, calculate: 5.7.1 The input kVA 5.7.2 The active component of the current | (4) (3) [30] |
| --- | --- |

| QUESTION 6 SEMI-CONDUCTORS - Briefly explain the difference between a silicon controlled rectifier and a conventional diode. 6.2 Explain by means of neatly labelled sketches and brief descriptions the basic construction and functional operation of a silicon controlled rectifier (SCR). | (3) (12) [15] |
| --- | --- |

| QUESTION 7 AMPLIFIERS |  |
| --- | --- |

| IC (mA) 15 10 5 0 2 4 6 8 10 12 14 VCE IB = 120 μA IB = 100 μA IB = 80 μA IB = 60 μA IB = 40 μA IB = 20 μA IB = 0 (Volts) Figure 7.1: Loadline |  |
| --- | --- |

| 7.1 Figure 7.1 illustrates the loadline of a certain amplifier. If we assume the amplifier employs class A amplification, determine 7.1.1 the supply voltage for this circuit. 7.1.2 the value of Rc. 7.1.3 the value of IB at the quiescent point. | (2) (3) (2) |
| --- | --- |

| 7.2 Answer the questions that follow with reference to Figure 7.2. |  |
| --- | --- |

| +VCC RB1 CE1 RB4 RB2 RC1 RB3 RC2 C2 CE2 C3 C1 Q1 Q2 + + Input Output Figure 7.2 |  |
| --- | --- |

| - Identify the circuit. - What is the purpose of each of the following? - RB1 and RB2. Indicate why this is necessary. - CE1 - C1 and C2 | (2) (4) (2) (2) |
| --- | --- |

| 7.3 Calculate the gain in dB’š if a 2W input signal results in a 6W output signal. | (3) |
| --- | --- |

| 7.4 Describe the term positive feedback. | (2) |
| --- | --- |

| 7.5 Answer the questions that follow with reference to Figure 7.3. |  |
| --- | --- |

| +VCC R1 T1 TF2 T2 TF1 R2 out in # C Figure 7.3 |  |
| --- | --- |

| - Identify the circuit in Figure 7.3. - Where is this type of amplifier generally used? - What is the function of TF? - What is the function of R2? - State TWO functions of TF2. | (2) (2) (2) (2) (2) [32] |
| --- | --- |

| QUESTION 8 SWITCHING AND CONTROL CIRCUITS |  |
| --- | --- |

| 8.1 Briefly explain the term regulated power supply. | (3) |
| --- | --- |

| 8.2 Answer the questions that follow with reference to Figure 8.1. |  |
| --- | --- |

| Vr1 R1 R2 RL T1 Vz Vin VBE Dz Vout VCE Figure 8.1 |  |
| --- | --- |

| - Identify the circuit in Figure 8.1 - Fully describe how the circuit will compensate for a reduction in the value of RL. | (2) (10) |
| --- | --- |

| 8.3 Draw a neat labelled circuit diagram of a 180º (full wave) phase control circuit to control the power of an electric motor. Explain what influence the capacitor value has on the working of the circuit. | (10) [25] |
| --- | --- |

| QUESTION 9 OSCILLATORS |  |
| --- | --- |

| 9.1 Answer the questions that follow with reference to Figure 9.1. |  |
| --- | --- |

| VCC L2 C1 L11 R2 T C2 R1 Y1 L12 C3 C4 Figure 9.1 |  |
| --- | --- |

| - Identify the circuit in Figure 9.1. - Explain the function of L2. - Explain how Y1 maintains a constant output frequency if the frequency of the tank circuit tends to increase. | (2) (2) (5) |
| --- | --- |

| 9.2 Name TWO differences between a bistable and an astable multivibrator. | (2) |
| --- | --- |

| 9.3 Explain the following terms with reference to oscillators: - Positive feedback - Tank circuit - Piezo electric effect | (2) (2) (2) [17] |
| --- | --- |

| QUESTION 10 APPLICATION OF OPERATIONAL AMPLIFIERS - Answer the questions that follow with reference to Figure 10.1. |  |
| --- | --- |

+ VCC

| R2 T2 V2 R3 A B RE R1 T1 V1 VEE R4 Figure 10.1 |  |
| --- | --- |

| 10.1.1 Identify the circuit in Figure 10.1. - What is the function of RE? - How will an increase in current through T1 influence the current throughT2? - In what mode will this circuit be when the output signals have the same amplitude and phase relationship? - Why does the circuit have a high degree of thermal stability? | (1) (2) (2) (1) (5) |
| --- | --- |

| 10.2 Name TWO characteristics of an operational amplifier. | (2) [13] |
| --- | --- |

| QUESTION 11 COMPUTER PRINCIPLES |  |
| --- | --- |

| 11.1 Give the Boolean expression of a half adder. | (2) |
| --- | --- |

| 11.2 Draw the logic diagram of an RS flip-flop and give the truth table thereof. | (10) |
| --- | --- |

| - An artisan uses THREE electric machines to manufacture furniture. The overload switch in the workshop cuts out on 121 Ampéres. Machine A uses 50A, machine B uses 70A and machine C uses 60 A. If the artisan uses more than 120A the power will be interrupted. 11.3.1 Draw a truth table for the combinations of the machines that would not interrupt the power supply. - Write down the Boolean expression for the functions in Question 11.3.1 and simplify the Boolean expression. - Draw a logical circuit for the simplified Boolean function. | (8) (7) (4) [31] |
| --- | --- |

| QUESTION 12 MEASURING INSTRUMENTS 12.1 Answer the questions that follow with reference to Figure 12.1. |  |
| --- | --- |

| display unit count gate control clocked pulse control circuit Schmitt trigger amplifier clocked pulse generator Schmitt trigger crystal osc input Figure 12.1 |  |
| --- | --- |

| - Identify the block diagram in Figure 12.1. 12.1.2 What is the function of the Schmitt trigger? 12.1.3 What is the function of the crystal oscillator? | (1) (2) (2) [5] |
| --- | --- |
| TOTAL: | 300 |

# FORMULES / FORMULAE

XL = 2πfL							IE = IB + IC

Tydkonstante / Time constant  t = RC		fr = 12πLC	en/and   1LC  −  R2L2

Z = R2++XL−Xc2				Cos Φ = PP skynbaar / apparent  LC - L2

VR = I . R

VL = I . XL

Ir = I sin Φ : Ia = I cos Φ							VC = I . XC

## Ster / Star

VP = VL3 : IP = IL					Z = VI		P 3 IL VL cos Φ

## Delta

VP = VL : IP = IL3					Z = LCR

I = VZ							f = 1T 		VPVS = NpNS = ISIP

ƒ = W2π							Ic = VceRL

Xc = 12πCF						Rendement / Efficiency = Uitset/OutputInset/Input

Cos Φ = RZ							Ic = V CL

S =Ns−NrNs							AF = Av / 1±BAv

β=IcIB							N = 10Log P2P1

Q = XLR							NpNs = ZpZs

N = Fp								Nr = Ns - S