GAUTENG DEPARTMENT OF EDUCATION SENIOR CERTIFICATE EXAMINATION TECHNIKA (ELECTRICAL) SG TIME: 3 hours MARKS: 200

| REQUIREMENTS: - Approved calculator and drawing instruments INSTRUCTIONS: - Answer ALL questions. - Sketches and diagrams should be clear and neat. - Formulae and calculations must be clear. - Formulae and calculations must be shown where indicated. - A list of formulae, which may be used where applicable, is given on the last page of this examination paper. |  |
| --- | --- |

| QUESTION 1 |  |
| --- | --- |

| - Briefly explain the meaning of the following terms: - Inductive reactance - Capacitive reactance - Impedance | (3) (3) (3) |
| --- | --- |

| - A resistance of 75 Ω and a capacitor of 50 μF are connected in parallel over a power supply of 220 V 50 Hz. Calculate the following: 1.2.1 Current through the resistance - Current through the capacitor - Supply current | (3) (6) (3) |
| --- | --- |

| 1.3 A coil with an inductance of 0,65 H and a resistance of 32 Ω is connected in series with a capacitor of 56 μF. The circuit is connected to a 150 V 50 Hz supply. Calculate the following: - Inductive reactance - Capacitive reactance - Impedance - Power of the circuit at 50 Hz - Draw a neatly labelled phasor diagram of the circuit at 50 Hz. | (3) (3) (3) (6) (4) [40] |
| --- | --- |

| QUESTION 2 SINGLE AND THREE-PHASE SYSTEMS |  |
| --- | --- |

| - Name the THREE colours, in the correct sequence, used to identify the three phases in a three-phase system. - Name THREE advantages that a three-phase system has over a single-phase system. - Explain, using a neatly labelled diagram, the relevance of line and phase voltages and currents in a star-coupled three-phase system. | (3) (3) (4) [10] |
| --- | --- |

| QUESTION 3 TRANSFORMERS |  |
| --- | --- |

| - A three-phase 6600/400 volt star-delta transformer has a rating of 250 kVA. The transformer has 100 windings on the secondary side. Calculate the following: 3.1.1 The number of primary windings - The maximum secondary phase current - The maximum output power if the load has a power factor of 0,8 lagging. | (3) (3) (3) |
| --- | --- |

| 3.2 Show, by means of a neatly labelled drawing, how a single-phase wattmeter can be connected to a high voltage, high current circuit using instrument transformers. 3.3 Briefly explain why it is not possible for a transformer to function when direct current is applied to it. | (8) (3) [20] |
| --- | --- |

| QUESTION 4 ALTERNATING CURRENT MOTORS |  |
| --- | --- |

| - Explain the purpose of the rotor bars and short circuit rings in the rotor of the induction-type alternating current motor. - Explain the following terms with reference to induction-type motors: 4.2.1 Synchronous speed 4.2.2 Rotor slip | (5) (2) (2) |
| --- | --- |

| - What is the function of the copper ring on the pole of the shaded-pole motor? - Explain why star/delta starters are used to start large three-phase motors under full load. | (3) (4) |
| --- | --- |

| - Draw a neatly labelled diagram of the following: 4.5.1 Capacitor-start motor 4.5.2 Squirrel-cage motor-three phase and coupled in delta | (6) (4) |
| --- | --- |

| - Briefly explain how a rotating magnetic field is obtained in the capacitor-start motor. | (4) [30] |
| --- | --- |

| QUESTION 5 SEMICONDUCTORS |  |
| --- | --- |

| - With the aid of a neatly labelled diagram, explain the principle of operation of a bipolar NPN transistor. - Draw the IEC symbols for the following components: 5.2.1 Silicon Controlled Rectifier - DIAC | (8) (1) (1) [10] |
| --- | --- |

| QUESTION 6 AMPLIFIERS |  |
| --- | --- |

| - Explain, using the relevant waveforms, the difference between class A, class B and class C amplification. - Name THREE advantages of negative feedback in relation to amplifiers. - Explain the principle of operation of the push-pull amplifier illustrated in Figure 6.3. | (6) (3) (7) |
| --- | --- |

| Figure 6.3 Push-Pull Amplifier |  |
| --- | --- |

| - Draw a neatly labelled diagram of the representative frequency curve of a transformer-coupled amplifier. | (4) [20] |
| --- | --- |

| QUESTION 7 SWITCHING AND CONTROL CIRCUITS |  |
| --- | --- |

| - Explain the functional principle of operation of the zener diode in relation to an increase in voltage over the diode and a decrease of voltage over the diode. - Explain the following terms in relation to Silicon-Controlled Rectifiers: 7.2.1 Holding current - Forward conducting voltage - Trigger pulse - Peak inverse voltage | (8) (3) (3) (3) (3) [20] |
| --- | --- |

| QUESTION 8 OPERATIONAL AMPLIFIERS |  |
| --- | --- |

| - Name FOUR qualities of an ideal operational amplifier. - Draw the output waveform of an operational amplifier differentiator circuit if a triangular waveform is applied to the input of the circuit. - Name FOUR items that should be included in the modern first aid kit. | (4) (2) (4) [10] |
| --- | --- |

| QUESTION 9 OSCILLATORS |  |
| --- | --- |

| 9.1 Identify the circuit in Figure 9.1 |  |
| --- | --- |

|  | (1) |
| --- | --- |

| Figure 9.1 |  |
| --- | --- |

| - Briefly explain the piezo-electrical principle. Make use of neatly labelled diagrams where necessary. - What is the purpose of the crystal in the feedback circuit of the crystal-controlled Hartley oscillator? | (6) (3) [10] |
| --- | --- |

| QUESTION 10 COMPUTER PRINCIPLES |  |
| --- | --- |

| 10.1 Convert the following product-of-the-sum expression to a sum-of-the-product expression. | (3) |
| --- | --- |

| (A + B) (B + C) (A + C) |  |
| --- | --- |

| - Name ONE associative Boolean Algebraic rule 10.3 Draw a neatly labelled logic circuit diagram of the following Boolean expression: | (1) (5) |
| --- | --- |

| [(A.B)+(A.C)]+D = X |  |
| --- | --- |

| 10.4 Name TWO precautionary measures to be taken when handling C-MOS logic devices. | (2) |
| --- | --- |

| 10.5 Set up a truth table for the RTL-gate in Figure 10.5. Draw a neatly labelled logic circuit diagram for this gate. | (5) |
| --- | --- |

| Figure 10.5 RTL-Gate |  |
| --- | --- |

| 10.6 Name THREE possible causes of fire in the workshop. | (3) |
| --- | --- |

| - Name the type of fire extinguisher that should be used in an electrical workshop. | (1) [20] |
| --- | --- |

| QUESTION 11 MEASURING INSTRUMENTS |  |
| --- | --- |

| - Name THREE uses of the multimeter. - Name THREE precautionary measures to be taken when working with an oscilloscope. - Is it possible to contract the HI-virus when working under normal conditions in the same workshop as an HIV-positive person? Motivate your answer. - What does the term creeping refer to in relation to single-phase energy meters? TOTAL: | (3) (3) (2) (2) [10] 200 |
| --- | --- |

| FORMULAE Time Constant t = RC and Star apparent |  |  |
| --- | --- | --- |