# GAUTENG DEPARTMENT OF EDUCATION # SENIOR CERTIFICATE EXAMINATION TECHNIKA (ELECTRONICS) SG TIME: 3 hours MARKS: 200

| INSTRUCTIONS: - Answer ALL questions. - An approved pocket calculator may be used. - Answers must be correctly numbered. - All sketches and diagrams must be large, neat and labelled. - Writing must be neat and legible. - A formulae sheet is included. |  |
| --- | --- |

| QUESTION 1 ELECTRICAL CURRENT THEORY |  |
| --- | --- |

| ##### A parallel circuit consists of a 9 Ω resistor, an inductor with a reactance of 12 Ω and a capacitor with a reactance of 16 Ω. These components are connected to a ##### 100 Volt /80 Hz supply. 1.1 Calculate the current through each branch. 1.2 Calculate the reactive current. 1.3 Calculate the total current. 1.4 Draw the phasor diagram. 1.5 Calculate the phase angle. 1.6 Calculate the impedance of the circuit. | (9) (3) (3) (6) (3) (3) [27] |
| --- | --- |

| #### QUESTION 2 SEMICONDUCTOR DEVICES Sketch the symbols of the following components and include inscriptions: 2.1 NPN-transistor 2.2 N-channel JFET 2.3 Darlington transistor | (3) (3) (4) [10] |
| --- | --- |

| # QUESTION 3 SWITCHING AND CONTROL CIRCUITS - Make use of two transistors and explain with the aid of a sketch and inscriptions the construction of a SCR. - Draw a circuit diagram to explain how you would use this SCR to switch on a globe supplied from a dc voltage of 10 V. - Name THREE requirements necessary for a SCR to function correctly. - Use a NPN transistor and explain by means of a circuit diagram how you would switch on a 100 W globe from an ac voltage of 220 V by making use of a relay. The voltage on the coil of the relay must be 10 V dc. - What will the voltage drop across a zener diode be when it is forward biased? | (5) (5) (3) (6) (1) [20] |
| --- | --- |

| # QUESTION 4 #### AMPLIFIERS 4.1 Make use of a graph and calculations to determine the load line of a common emitter amplifier using a 1kΩ and 5kΩ load resistor respectively. The supply voltage is 100V dc. (The circuit has no emitter resistor.) | (15) |
| --- | --- |

| # 4.2 A 741 operational inverting amplifier amplifies a signal of 1 mV to a 100 mV. The input impedance is 1kΩ. Calculate the feedback resistor and draw the circuit diagram with inscriptions. | (17) |
| --- | --- |

| # 4.3 Draw a circuit diagram of a symmetrical class AB push-pull amplifier. | (12) [44] |
| --- | --- |

| # QUESTION 5 #### VOLTAGE REGULATION 5.1 Draw a circuit diagram of a series voltage regulator with a load resistor, and calculate the value of the resistor in series with the zener diode if the input voltage is 20 V and the output voltage is 10 V. Make use of a 400 mW zener diode. (Accept that no load is connected to the circuit when calculations are made.) What will the loss in watts be across the transistor if the load current is 1 A? | [20] |
| --- | --- |

| # QUESTION 6 #### MULTIVIBRATORS 6.1 A bi-stable multivibrator makes use of an operational amplifier. Draw a circuit diagram of the multivibrator. | [10] |
| --- | --- |

| # QUESTION 7 #### COMPUTER PRINCIPLES - Sketch the symbols of each of the following logic gates: - AND – gate - OR – gate - NOT – gate - Sketch the circuit diagram of the following Boolean expression, and complete a truth table to show the outputs. (Ā.B) + C = X - Sketch the block diagram of a full adder | (6) (17) (5) [28] |
| --- | --- |

| # QUESTION 8 #### OSCILLATORS 8.1 Draw a labelled circuit diagram of a Hartley oscillator. | [14] |
| --- | --- |

| # QUESTION 9 #### INFORMATION TRANSFER 9.1 Make use of a block diagram and explain the construction of a fibre optic telephone communication system. | [10] |
| --- | --- |

| # QUESTION 10 #### MEASURING INSTRUMENTS # 10.1 Explain by means of a block diagram the construction of a simple digital multimeter. | [6] |  |
| --- | --- | --- |

| #### QUESTION 11 SAFETY PRECAUTIONS - Name FOUR unsafe acts in a workshop. - A person has been exposed to an electrical shock and has stopped breathing. Explain how you would perform mouth-to-mouth resuscitation. - A person accidentally cuts himself/herself in the workshop and you have to treat the wound. What precautions would you take against possible AIDS infection? | (4) (5) (2) [11] |
| --- | --- |

TOTAL:               200