GAUTENG DEPARTMENT OF EDUCATION SENIOR CERTIFICATE EXAMINATION TECHNIKA (ELECTRONICS) HG TIME: 3 hours MARKS: 300

| INSTRUCTIONS: - Answer ALL the questions. - Sketches and diagrams must be large, neat and labelled. - All calculations must be shown. - Answers must be clearly numbered. - A formula sheet (pages 10 – 13) is provided at the end of the paper. |  |
| --- | --- |

| QUESTION 1 ELECTRIC CURRENT THEORY |  |
| --- | --- |

| 1.1 A parallel circuit consists of a coil with an inductance of 9 µH, a capacitor with a capacitance of 100 pF and a resistor with a resistance of 2 kΩ. The circuit is supplied from a 100 Volt/5 MHz supply. |  |
| --- | --- |

| Calculate: |  |
| --- | --- |

| - The current in each branch - The total current in the circuit - The phase angle and sketch a fully labelled phasor diagram representing the currents. - The impedance of the circuit | (15) (3) (9) (3) |
| --- | --- |

| 1.2 Briefly explain the term impedance matching and name ONE application thereof. | (4) |
| --- | --- |

| 1.3 Calculate the turns ratio required for an impedance matching transformer to satisfy the requirements of a loudspeaker with an 8Ω voice coil. The transistor needs a 500Ω load for maximum power transfer. | (4) [38] |
| --- | --- |

| QUESTION 2 SEMICONDUCTOR DEVICES |  |
| --- | --- |

| 2.1 Identify the following electronic components with reference to the electronic circuit diagram in Figure 2.1. Example: R225 is a 3K3Ω resistor. |  |
| --- | --- |

| - T113 - R235 - D107 - L122 - C188 | (2) (3) (1) (2) (2) |
| --- | --- |

| ### Blue |  |
| --- | --- |

| Figure 2.1 |  |
| --- | --- |

| 2.2 The field effect transistor (FET) Has been used since 1952 as a semiconductor device. Explain, by means of neatly labelled sketches and brief descriptions, the basic construction and functional operation of a field effect transistor (FET). | (12) |
| --- | --- |

| 2.3 Name FOUR precautions that must be taken into account when handling CMOS devices. | (4) |
| --- | --- |

| 2.4 To identify the pins of integrated circuits (IC), they are each numbered from number one upwards. Sketch the integrated circuit of Figure 2.2 in your answer book and number the pins. |  |
| --- | --- |

| Figure 2.2 The dual in line package | (2) [28] |
| --- | --- |

| QUESTION 3 AMPLIFIERS |  |
| --- | --- |

| 3.1 Determine by means of calculations and a neatly labelled graph the load line of the circuit shown in Figure 3.1. | (9) |
| --- | --- |

| Calculate: |  |
| --- | --- |

| - The base current (Ib) - The collector current (Ic) if the gain (ß) of the transistor is given as 120 | (4) (4) |
| --- | --- |

| Vcc=12 Volt Rc=800 Ω 120 KΩ Rb1 C2 C1 |  |
| --- | --- |

| Figure 3.1 |  |
| --- | --- |

| 3.2 A 741 operational amplifier is connected in the inventing mode. It amplifies a signal of 1mV to 100mV. The input impedance is 1kΩ. |  |
| --- | --- |

| - Draw a neatly labelled diagram of the circuit. - Determine the value of the feedback resistor by means of calculations. | (10) (7) [34] |
| --- | --- |

| QUESTION 4 SWITCHING AND CONTROL CIRCUITS |  |
| --- | --- |

| 4.1 Draw a neatly labelled circuit diagram of a fixed voltage series regulator with the following values: |  |
| --- | --- |

| Input voltage = 20 Volt Output voltage = 10 Volt Power of the zener diode = 400 mWatt | (8) |
| --- | --- |

| 4.1.1 Calculate the value of the resistor (Rs) in series with the zener diode. | (13) |
| --- | --- |

| 4.2 Figure 4.1 illustrates the electronic circuit of a brushless fan commonly found in computers. The motors within these fans generally consist of an outer revolving armature containing permanent magnets surrounding a static inner coil assembly. The coils are switched sequentially to produce the rotation by an internal electronic circuit as shown in Figure 4.1. Explain the working principle of this circuit. | (12) |
| --- | --- |

| Figure 4.1 Speed controlled circuit diagram for a brushless fan |  |
| --- | --- |

| 4.3 Clippers and clampers are diode wave shaping circuits transmitting parts of waveforms and suppressing others to a predetermined value. Find the output voltage wave shape for the inputs shown in Figure 4.2. (Only sketch the output voltage wave shape in your answer book.) |  |
| --- | --- |

D

R

Vin

Vout

O

| (a) V | (2) |
| --- | --- |

| D V O 6 V R Vout Vin (b) | (2) |
| --- | --- |

| Vout Vin R D V O (c) | (2) |
| --- | --- |

| Figure 4.2 Wave shaping circuits |  |
| --- | --- |

| 4.4 Explain the working principle of ANY electronic experiment or model that you have built/designed this year. Take note that your explanation should include a neatly labelled circuit diagram or block diagram with a brief description. All wave forms, where applicable, should be included. | (11) [50] |
| --- | --- |

| QUESTION 5 OSCILLATORS |  |
| --- | --- |

| 5.1 Explain what an electronic oscillator circuit is. | (3) |
| --- | --- |

| 5.2 Explain with the aid of a neatly labelled characteristic curve and a brief description of the characteristic curve, the operating principle of a unijunction transistor (UJT). | (15) [18] |
| --- | --- |

| QUESTION 6 COMPUTER PRINCIPLES |  |
| --- | --- |

| 6.1 Design a NOR gate network for the following Boolean expression (A, B, C and D are direct gate inputs.): |  |
| --- | --- |

| F =(A− + B)(C +D−) | (7) |
| --- | --- |

| 6.2 Simplify the following Boolean expression: |  |
| --- | --- |

| F = [AB− (C+ BD) + A−B−] C | (7) |
| --- | --- |

| 6.3 A hospital requests you to design an electronic circuit to monitor the condition of a patient in the I.C.U. An alarm must sound when the following conditions occur: |  |
| --- | --- |

| - The blood pressure of a person is not normal or - The temperature and blood pressure are not normal or - The temperature and pulse are not normal |  |
| --- | --- |

| 6.3.1 Design the logic circuit to satisfy the request. | (15) |
| --- | --- |

| 6.4 Refer to Figure 6.1 and draw a truth table that will indicate when LED 1 and LED 2 are switched on. Note that the symbol for the logic gates is the American equivalent for a NOR gate. | (9) |
| --- | --- |

| Figure 6.1 Electronic circuit |  |
| --- | --- |

| 6.4.1 Name the circuit diagram of Figure 6.1. | (1) |
| --- | --- |

| 6.5 Illustrate by means of a neatly labelled block diagram of a full adder circuit how the following two binary numbers are added: |  |
| --- | --- |

| 1001 + 0011 ____ | (16) [55] |
| --- | --- |

| QUESTION 7 INFORMATION TRANSFER |  |
| --- | --- |

| 7.1 Fibre optic cables are manufactured in different sizes and construction. Describe the basic construction of a fibre optic cable by means of a neatly labelled sketch. | (7) |
| --- | --- |

| 7.2 Explain the importance of interfacing (joining) between fibre and cable ends. | (6) |
| --- | --- |

| 7.3 Discuss the advantages of using fibre optic cables. | (5) |
| --- | --- |

| 7.4 Explain by means of a neatly labelled block diagram the working principle of a typical fibre optic communication system. | (13) |
| --- | --- |

| 7.5 Explain what a modem is and how it is used with reference to computers. | (5) [36] |
| --- | --- |

| QUESTION 8 MEASURING INSTRUMENTS |  |
| --- | --- |

| 8.1 Explain the working principle of a digital multi-meter by means of a neatly, labelled block diagram. | (6) |
| --- | --- |

| 8.2 List FIVE advantages a digital multi-meter has compared to an analogue meter. | (5) |
| --- | --- |

| 8.3 Determine the reading of the multi-meter for the switch illustrated in Figure 8.1. | (2) |
| --- | --- |

| Figure 8.1 Multi-meter reading |  |
| --- | --- |

| 8.4 A sinusoidal waveform is observed on the screen of an oscilloscope as having a peak amplitude of 3 cm (divisions). If the CRO vertical sensitivity setting is set at 10 V/cm (divisions) and a “time ten” (10) probe is used, calculate the RMS value of the voltage waveform. | (6) |
| --- | --- |

| 8.5 Explain the purpose of the X and Y deflection plates of an oscilloscope. | (6) [25] |
| --- | --- |

| QUESTION 9 SAFETY PRECAUTIONS |  |
| --- | --- |

| 9.1 Name FIVE safety checks that should be done before using portable electrical equipment from a socket outlet. | (5) |
| --- | --- |

| 9.2 Name FIVE precautions that should be taken into account when handling a multimeter. | (5) |
| --- | --- |

| 9.3 Explain why foam fire extinguishers should not be used on electrical fires. | (2) |
| --- | --- |

| 9.4 Explain how Aids can be spread from one person to another and explain how you would protect yourself against the possible infection of HIV. | (4) [16] |
| --- | --- |

| TOTAL: | 300 |
| --- | --- |