GAUTENG DEPARTMENT OF EDUCATION SENIOR CERTIFICATE EXAMINATION TECHNIKA (ELECTRONICS) SG TIME: 3 hours MARKS: 200

| INSTRUCTIONS: - Answer ALL the questions. - Sketches and diagrams must be large, neat and labelled. - All calculations must be shown. - Answers must be clearly numbered. - An approved pocket calculator may be used. |  |
| --- | --- |

| QUESTION 1 ELECTRIC CURRENT THEORY - The following three components are connected in series: R = 20Ω, L = 0, 1H and C = 15μF. The supply voltage is 100V/100Hz. Calculate the following: - The voltage drop across each component - The total current through the circuit - The phase angle (Draw a labelled phasor diagram.) - Determine the minimum and maximum values of a variable capacitor connected in parallel with a 10μH coil that resonates between a frequency of 500kHz and 1Mhz. 1.3 A 0,1mH coil and a resistor of 8Ω are connected across an alternating voltage with a frequency of 10kHz. Calculate the Q-factor of the circuit. | (18) (3) (8) (7) (6) [42] |
| --- | --- |

| QUESTION 2 SEMICONDUCTOR DEVICES - Identify the following components in Figure 1. - T1 - D1 - R1 - R2 - L1 2.1.6 C1 | (6) |
| --- | --- |

| 470Ω R2 L1 10µF 100K 1N4001 D1 24K T2 91Ω 1K 4K7 10K 5K5 T1 10µF 8µF 75K C1 7K2 125V R1 Output Input 250m H 1N4002 30K |  |
| --- | --- |

Figure 1

| - Name FOUR requirements necessary for an SCR to function correctly. - Once conducting, an SCR continues to operate from a 12V/DC supply regardless of the voltage on the gate. The SCR must switch on a light. Draw a labelled circuit diagram of such a device, and show how one can switch off the SCR. - Write down the standard colour code you would use to determine the value and tolerance of any resistor. | (8) (8) (12) [34] |
| --- | --- |

| QUESTION 3 AMPLIFIERS - Draw a labelled circuit diagram of a symmetrical push-pull pair-class AB amplifier. - Make use of an NPN transistor, capacitors and resistors and explain with the aid of a circuit diagram how an inverting amplifier amplifies an input signal. Draw the input and output signals. - Make use of a graph and calculations to determine the load line of a common emitter amplifier using a 1 kΩ and 5kΩ load resistor respectively. The supply voltage is a 100V/DC. (The circuit has no emitter resistor.) - Draw a labelled circuit diagram of an operational summing amplifier. - What would the summing amplifier most commonly be used for? | (10) (8) (15) (8) (1) [42] |
| --- | --- |

| QUESTION 4 SWITCHING AND CONTROL CIRCUITS - Draw a labelled circuit diagram of - a series voltage regulator. - a shunt voltage regulator. - Make use of transistors and draw a labelled circuit diagram of an astable multivibrator. | (12) (10) |
| --- | --- |

| 4.3 What is the circuit diagram in Figure 2 called? | (1) |
| --- | --- |

+vCC R4 R2 output B output A R1 Y3 R2 T2

Figure 2

| 4.4 Suppose T1 in Figure 2 is on after the circuit has been switched on, explain how T1 reacted first. | (4) [27] |
| --- | --- |

| QUESTION 5 OSCILLATORS 5.1 Draw a labelled circuit diagram of a Hartley oscillator. | (12) [12] |
| --- | --- |

| QUESTION 6 COMPUTER PRINCIPLES - Sketch the following gates and complete a truth table for each. - NAND - NOR - OR | (12) [12] |
| --- | --- |

| QUESTION 7 INFORMATION TRANSFER 7.1 With the aid of a labelled block diagram explain the construction of a fibre optic telephone communication system. | (10) [10] |
| --- | --- |

| QUESTION 8 MEASURING INSTRUMENTS - Determine the RMS voltage and the frequency of the wave shown in Figure 3 when the Volt/div switch is set on 1 Volt/div and the sec/div switch is set on 5ms/div. | (10) |
| --- | --- |

100

90

10

0%

| Figure 3 | [10] |
| --- | --- |

| QUESTION 9 SAFETY PRECAUTIONS - Name FOUR unsafe acts in a workshop. - Explain how you would perform mouth to mouth resuscitation in order to save the life of a person who has been exposed to an electrical shock in the workshop and has stopped breathing. - Say whether the following statements are TRUE or FALSE. - By using the same tools as an HIV positive person, one can easily contract this virus. - One never has to check electrical cords of electrical hand equipment. TOTAL: | (4) (5) (1) (1) [11] 200 |
| --- | --- |