| GAUTENG DEPARTMENT OF EDUCATION SENIOR CERTIFICATE EXAMINATION TECHNIKA (MECHANICAL) SG TIME: 3 hours MARKS: 200 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| REQUIREMENTS: Calculator and drawing instruments. INSTRUCTIONS: - Question 1 is COMPULSORY. - Answer Question 1 and any FOUR other questions. - Answer Questions 1.1 to 1.5 on the answer sheet on the inside cover of your answer book. - Information pages are provided (pages 9-12). |  |  |  |  |  |  |
| QUESTION 1 COMPULSORY Choose the correct answer out of each of the following and draw a cross (X) over the appropriate letter next to the respective question number (1.1 to 1.5) on the answer sheet, on the inside cover of your answer book. - Which of the following is a vector quantity? |  |  |  |  |  |  |
|  | A. | Energy | B. | Momentum |  |  |
|  | C. | Mass | D. | Time |  |  |
|  | E. | Distance |  |  |  |  |

| 1.2 The displacement divided by the time interval is called: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A. | Speed | B. | Average speed |  |  |
|  | C. | Average velocity | D. | Instant velocity |  |  |
|  | E. | Acceleration |  |  |  |  |

| 1.3 The power or tempo of work done by a machine, if 60 000 J of work is done per minute, is |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A. | 1 000 W. | B. | 36 000 000 W. |  |  |
|  | C. | 60 000 W. | D. | 6 W. |  |  |
|  | E. | 6 000 W. |  |  |  |  |

| 1.4 Two forces of 3 N and 5 N respectively act on the same point. The magnitude of their resultant is 2 N when the work lines of the forces are at an angle of |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | A. | Oo. | B. | 45o. |  |  |
|  | C. | 90o. | D. | 180o. |  |  |
|  | E. | 360o. |  |  |  |  |

| - According to Boyle’s Law for a mass of gas at constant temperature, - the pressure is constant. - the volume increases. - the pressure is directly proportional to the volume. - the pressure is inversely proportional to the volume. - the volume is inversely proportional to the pressure | 5x2=(10) |
| --- | --- |

| Answer the following questions in full: - State THREE advantages and THREE disadvantages of V-belts over flat belts. - When the blade of a hacksaw is tightened, the blade is under stress. State the type of stress. - Distinguish between speed and velocity. - Define the term resultant. - Draw the symbol which is used to represent each of the following components in an electric circuit: - A transistor - A rectifier diode | (6) (1) (2) (4) (3) (3) |
| --- | --- |

| - Define a radian. - Define torque. - State THREE facilities necessary to ensure the proper hygiene of factory personnel. | (4) (4) (3) [40] |
| --- | --- |

| QUESTION 2 - Ninety nine (99) teeth must be milled onto a spur gear. The dividing head ratio is 40:1. - Calculate the indexing required. (Choose 100 divisions). - Calculate the change wheels required. - Determine the direction of rotation of the index plate. | (3) (5) (1) |
| --- | --- |

| - State FOUR aspects with regard to safe working conditions for employees, which are important when designing a factory building. - Why is good industrial housekeeping important in a factory? - What is an employee’s responsibility with regard to industrial diseases? - The abuse of alcohol and drugs is a social evil of our time. State FIVE detrimental consequences of such abuse for an individual’s work situation and/or family. - Briefly describe how a two start 5 mm pitch square thread is cut on a lathe using the compound slide method of thread cutting. - State THREE reasons why surface roughness occurs when parts are machined. - The surface of a machine component has to be finished by lapping to a finish between 0,05 and 0,10 m. Sketch the symbol that will show this finish on a drawing. | (4) (4) (4) (4) (9) (3) (3) [40] |
| --- | --- |

| QUESTION 3 ### Figure 1 - 3.1.1 Sketch a complete iron carbon equilibrium diagram for types of steel. Indicate the temperature between 0oC and 1000oC, and the carbon content between 0% and 1,4%. - Determine, with the aid of the above-mentioned diagram, the structure of steel with a carbon content of 0,4% at a temperature of 800oC. - Name THREE space lattice types with reference to metals and state the atomic number of each. - Provide TWO examples of each of the following types of tests: - Machinable tests - Destructive tests - Non-destructive tests - State FOUR factors which influence the change in the structure of steel during the arc-welding process. | (13) (2) (12) (3) (3) (3) (4) [40] |
| --- | --- |
| QUESTION 4 50N 45º 75º 45º 30N 100N - Determine graphically the magnitude of the resultant force of the system of concurrent forces shown in the figure. | (8) |

Figure 1

| A casting with a mass of 5 000 kg rests on a horizontal surface. The coefficient of friction between the surfaces is 0,35. Calculate the force (F) required to pull the casting if the force is inclined at 20o to the horizontal. F 20 ° ° ° 5000 KG Figure 2 Figure 2 4.3 Draw a neat, labelled sketch of a typical stress-strain graph that is obtained when mild steel is subjected to a destructive tensile test. | (12) (7) |
| --- | --- |

| - Calculate the included angle of a M50 V screw thread with a pitch of 6 mm. The distance over the large and the small measuring wires shows a total difference of 3,56 mm. Given: Maximum diameter of measuring wires = 0,76 P Minimum diameter of measuring wires = 0,54 P | (6) |
| --- | --- |

| - Use the table of primary selection of fits, and state the following: - The limits of size for a 34 H7 – n6 hole shaft combination 4.5.2 The type of fit | (6) (1) [40] |
| --- | --- |

| QUESTION 5 - The ultimate stress for a particular type of steel used to manufacture bolts is 500 MPa. Calculate the - load in newton to fracture a M6-bolt at the root diameter of the thread if the root diameter of the thread is 4,55 mm. - safe working stress for the bolt where a safety factor of 7 is used. - An engine with a mass of 540 kg is lifted from ground level to a height of 1,2 metres with the aid of a crane. Calculate - the force required to lift the engine with no acceleration. - the work done to lift the engine to a height of 1,2 metres. | (6) (3) (4) (4) |
| --- | --- |

| Draw a neat sectional sketch of a single plate helical spring dry clutch. Clearly indicate the following components: Figure 3 - Flywheel - Pressure plate - Pressure springs (helical springs) - Clutch plate - Clutch release levers (clutch release fingers) - Input shaft - Output shaft - Thrust bearing - Pilot bearing - What type of universal joint is used in a motor vehicle with front wheel drive? Explain why. | (20) (3) [40] |
| --- | --- |
| QUESTION 6 - An electric motor turns with a speed of 1 000 rpm. It drives a pulley wheel with a diameter of 1 200 mm that fits onto the machine spindle by means of a drive belt at a speed of 200 rpm. What is the size of the pulley wheel that is mounted on the machine spindle? Please note: This kind of drive is known as a simple drive. Figure 3 - A drive gear with 50 teeth meshes into an idler with 70 teeth which rotates at 210 rpm. The idler in turn, meshes into a driven gear that turns at 100 rpm. Calculate: - the rotation frequency of the drive gear. - the number of teeth of the driven gear. - the speed ratio of the gear train. | (6) (4) (7) (8) |

| - A crane, with a drum diameter of 550 mm and a rotation frequency of 50 r/min, lifts a load of 5 kN with the aid of a rope. Calculate the - torque required. - speed of the rope in m.s –1. - work done per minute. - power required to lift the load. - efficiency of the crane if it is powered by a 15 kW electric motor. | (3) (3) (3) (4) (2) [40] |
| --- | --- |
| QUESTION 7 - The following is the data for a four cyclinder four stroke internal combustion engine: Mean effective pressure on piston = 1 200 kPa Stroke = 70 mm Bore (cylinder diameter) = 70 mm Revolutions per second = 52 Effective brake arm length = 900 mm Reading on scale = 9,4 kg Calculate the 7.1.1 indicated power in kW. 7.1.2 brake power in kW. 7.1.3 mechanical efficiency as a percentage. - Name EIGHT components which are used in a hydraulic system. | (9) (7) (4) (8) |

| - Calculate the width, thickness and length of a parallel key used to secure a pulley on a 100 mm diameter shaft. - State TWO characteristics and TWO uses, of each of the following carbon compounds: - Alkane - Alkene TOTAL: | (4) (4) (4) [40] 200 |
| --- | --- |