| GAUTENG DEPARTMENT OF EDUCATION SENIOR CERTIFICATE EXAMINATION WELDING AND METALWORKING SG TIME: 3 hours MARKS: 200 |  |
| --- | --- |
| REQUIREMENTS: - A3 drawing paper - Drawing instruments - An approved pocket calculator INSTRUCTIONS: - You must answer FIVE questions: – Question 1 is COMPULSORY. – Answer any FOUR questions from Section B. - Number your answers in accordance with the question paper. |  |

| SECTION A QUESTION 1 COMPULSORY |  |
| --- | --- |

| - Each of the following questions has five possible answers, only ONE of which is correct. Write down the appropriate letter next to the relevant question number e.g. 1.1.1A. |  |
| --- | --- |

| 1.1.1 One of the workers slips, falls and hits his head against the work-bench and an open wound appears. How will you treat this person without contaminating yourself with the AIDS virus? - Immediately put an adhesive plaster on the wound. - Apply immediate pressure onto the wound with your hand. - Put on surgical gloves before touching the blood or the wound. - Clean the wound with a clean cloth and treat it. - Clean the wound with water. |  |
| --- | --- |

| - Which of the following properties are obtained when a metal is case-hardened by the heat-treatment process? - Hard and brittle - Hard and tough - The outer layer is hard and the core is tough. - The outer layer is soft and the core is hard. - The metal is not shock resistant. |  |
| --- | --- |

| - Elasticity is defined as the ability - to withstand abrasion. - to withstand shock loads. - of a metal to return to its orginal dimension after the load has been removed. - of metal to break easily. - to resist wear and abrasion |  |
| --- | --- |

| - Bad root-weld penetration is caused by - a current setting that is too high. - a root opening which is too small. - no or insufficient plate bevelling. - a root opening which is too small and an insufficient plate bevelling. - None of the aspects mentioned in A, B, C or D. |  |
| --- | --- |

| - The lower critical temperature as given in the iron-carbon equilibrium diagram, stays constant at a given temperature of - 780ºC. - 730ºC. - 720ºC. - 710ºC. - 750ºC. | 5x2=(10) |
| --- | --- |

| 1.2 Choose the correct answer from Column B to fit the information in Column A by writing only the letter of your choice next to the corresponding question number, e.g. 1.2.1 A. |  |
| --- | --- |

| COLUMN A | COLUMN B |
| --- | --- |
| 1.2.1 Critical point AC3 | A. 1012 |
| 1.2.2 This is an internal force in material resisting a load. | B. Wrong electrode |
| 1.2.3 Overhead costs | C. N/m2 |
| 1.2.4 Tera | D. Boring machine |
| 1.2.5 Template loft | E. Tensile stress |
| 1.2.6 Pa | F. 109 |
| 1.2.7 Distributed load | G. Rent of premises |
| 1.2.8 This is an internal force present in material when an external force is applied. | H. Austenite structure |
| 1.2.9 Weld spatter | I. Beams |
| 1.2.10 Giga | J. Stress |

(10)

| 1.3 Indicate whether each of the statements below is TRUE or FALSE. Write only the number of the question and TRUE/FALSE in your answer book. |  |
| --- | --- |

| 1.3.1 Bad root penetration is caused by using a current setting that is too high. |  |
| --- | --- |

| 1.3.2 The presence of spatter on top of a weld is caused mainly by an arc length that is too high . |  |
| --- | --- |

| 1.3.3 The x-ray testing method (that is used mainly in testing welded joints) has certain advantages because it can be easily installed. |  |
| --- | --- |

| 1.3.4 The ultrasonic testing method (which is usually used in determining weld defects) entails the use of transmitters to create sound waves. |  |
| --- | --- |

| 1.3.5 The non-destructive method of testing welded joints entails the breaking of a test piece to enable easy location of welding defects. |  |
| --- | --- |

| 1.3.6 The properties of hardening is that the metal has become extremely hard and brittle. |  |
| --- | --- |

| 1.3.7 Normalising is used especially on steel where the steel was subjected to internal stresses. |  |
| --- | --- |

| 1.3.8 The properties of case-hardening are that the whole structure of steel becomes hard and brittle. |  |
| --- | --- |

| 1.3.9 Tempering can only be carried out on brass. |  |
| --- | --- |

| 1.3.10 Lines on the graph paper that correspond to the upper and lower critical temperature intersect each other where the steel has a carbon content of 0,75%. |  |
| --- | --- |

| 1.3.11 Rays that are formed by the X-ray machine consist of electro-magnetic rays of shortwave length and can penetrate any solid mass of steel. |  |
| --- | --- |

| 1.3.12 With reference to the iron-carbon equilibrium diagram, carbon steels heated above the upper critical temperature mainly contain ferrite and perlite. |  |
| --- | --- |

| 1.3.13 Grinding machines can only grind mild steel. |  |
| --- | --- |

| 1.3.14 A safety factor of not less than three must be obtained with ladders and scaffolding. |  |
| --- | --- |

| 1.3.15 Mild steel cannot be hardened because it is ductile and malleable. | (15) |
| --- | --- |

| 1.4 State THREE characteristics which the floor in a template loft should have. | (3) |
| --- | --- |

| 1.5 State TWO advantages of castellated beams. | (2) [40] |
| --- | --- |

| TOTAL FOR SECTION A: | [40] |
| --- | --- |

| SECTION B Answer any FOUR questions from this section. |  |
| --- | --- |

| QUESTION 2 Figure 1 shows a line diagram of a steel framework. The framework is welded and consists of single angle profiles without gusset plates. The total time taken to manufacture one framework is 7 hours, at a tariff of R58,50 per hour, while the overhead costs are calculated at 75% of the labour costs. The cost of the angle profiles of 60x60x6 mm with a mass of 4,7 kg/m length is R1,50 per kilogram and profiles of 50x50x6 mm with a mass of 4,47 kg/m length is R1,50 per kilogram. An amount of R25 is allocated for welding material. |  |
| --- | --- |

# B # A # D # E # F # C Figure 1

| - Copy and complete the undermentioned table in your answer book. Calcuate: 2.1.1 The cost of material - The labour costs - The overhead costs - The total cost of the completed framework. | (14) (3) (3) (2) |
| --- | --- |

| MARK | NUMBER REQUIRED | MATERIAL REQUIRED | MASS/M LENGTH | TARIFF PER KG | R C |
| --- | --- | --- | --- | --- | --- |
| A | 1 | 60x60x6mm 5,8 m long | 4,7 kg | 1,50 |  |
| B | 1 | 60x60x6mm 11,5 long | 4,7 kg | 1,50 |  |
| C | 1 | 60x60x6mm 10 m long | 4,7 kg | 1,50 |  |
| D | 1 | 50x50x6mm 5m long | 4,47 kg | 1,50 |  |
| E | 1 | 50x50x6mm 5m long | 4,47 kg | 1,50 |  |
| F | 1 | 50x50x6mm 2,5m long | 4,47 kg | 1,50 |  |
|  |  |  | Welding material costs |  |  |
|  |  |  | Material costs (Total) |  |  |

| 2.2 Draw neat sketches of the front elevation and the top view of a bolted column base, manufactured from the following components: ONE 300 mm by 200 mm rolled steel column FOUR 70 mm by 70 mm by 8 mm angle irons ONE base plate 600 mm by 600 mm by 20 mm thick TWO wing plates 12 mm thick Bolts need not be shown. | (9) |
| --- | --- |

| 2.3 Draw neat sketches to show how the following rolled sections are prepared for welding to form a T-joint: 2.3.1 I beam to I beam of different sizes 2.3.2 Channel iron to channel iron of different sizes. | (3) (3) |
| --- | --- |

| 2.4 Name the THREE types of templates that will be used to mark the positions of the holes on different steel profiles. | (3) [40] |
| --- | --- |

| QUESTION 3 |  |
| --- | --- |

| 3.1 Inspection during welding is very important for obtaining the best results. Name THREE important factors to take into account during welding. | (6) |
| --- | --- |

| 3.2 State FIVE defects which may occur in arc welding. | (5) |
| --- | --- |

| 3.3 What is the main purpose of the heat treatment process of hardening? | (3) |
| --- | --- |

| 3.4 Name SIX tools that are used in the template loft. | (6) |
| --- | --- |

| - Draw a fully detailed sketch of the carbon equilibrium diagram for carbon steel with 0% to 1,4% carbon content and a temperature of 500ºC to 1 000ºC. | (14) |
| --- | --- |

| 3.6 List TWO factors that will determine the choice of an electrode when welding. | (2) |
| --- | --- |

| 3.7 List FOUR safety precautions you would observe when welding using the electric arc welder. | (4) [40] |
| --- | --- |

| QUESTION 4 |  |
| --- | --- |

| - A tensile force of 60 kN causes a steel bar with a diamater of 36 mm and a length of 1,6 m to stretch 0,4 mm. 4.1.1 Indicate by means of calculations that the stress in the bar is 58,946 MPa - Indicate also by means of calculations that the strain is 0,25 x 10-3 . 4.1.3 Calculate the value of Youngs Modulus for this bar. | (10) (7) (7) |
| --- | --- |

| 4.2 Calculate the load to which a 40 mm squarebar is subjected if the stress is 5x106 Pa. | (9) |
| --- | --- |

| 4.3 Calculate the length of a bar if it shortens by 0,6 mm under a compression force. The strain is 0,8 x 10-3. | (7) |
| --- | --- |

| Formulas: ## Load______________ Cross-sectional area ## Deformation_________ Original length ## Stress _________ Strain Stress = Strain = Young’s Modulus (E) = | [40] |
| --- | --- |

| QUESTION 5 |  |
| --- | --- |

800 600 300 300 600 A B H J K C E D Figure 2

| Figure 2 shows the front view and the top view of a square to a square hopper, that has been turned. - Calculate the plan length of EK. - Calculate the true length of EK. - Calculate the plan length of CK. - Calculate the true length of CK. - Calculate the length of HK. | (4) (9) (9) (9) (9) |
| --- | --- |

| (Use the theorem of Pythagoras.) | [40] |
| --- | --- |

| QUESTION 6 |  |
| --- | --- |

5kN

B

C

4 kN

2 kN

G

45º

45º

H

F

K

A

D

E

30º

30º

30º

30º

3m

3m

3m

3m

LR = 6 kN

RR = 5 kN

### Figure 3

| 6.1 Indicate by means of calculation that the following reactions at supports L and R will read 6 kN and 5 kN respectively. | (4) |
| --- | --- |

| 6.2 Draw the space diagram of the framework to a scale of 1:100 and indicate the nature of the forces on it. | (9) |
| --- | --- |

| 6.3 Draw the force diagram to a scale of 1 kN = 10 mm. | (9) |
| --- | --- |

| 6.4 Determine the magnitude and nature of the forces in each component of the framework. Copy and complete the table on page 11 in your answer book. | (18) |
| --- | --- |

| MEMBER | FORCE - kN | NATURE |
| --- | --- | --- |
| AF |  |  |
| BG |  |  |
| CG |  |  |
| DK |  |  |
| GH |  |  |
| EK |  |  |
| EF |  |  |
| FH |  |  |
| HK |  |  |

[40]

| QUESTION 7 |  |
| --- | --- |

| 7.1 Figure 4 shows a beam, 6 metres long, supported at its ends. The beam carries one distributed load and two vertical point loads. | (10) |
| --- | --- |

| 7.1.1 Calculate the reaction forces at supports L and R. |  |
| --- | --- |

5 kN

4 kN

2 M 1 M 2 M 1 M 0,5 kN/m L R Figure 4

| 7.2 Figure 5 shows a cantilever beam 6 metres long which is firmly fixed at point A. The beam is subjected to three vertical point loads. |  |
| --- | --- |

10 kN 20 kN A B C P D 3m 3m 3m 3m 30 kN Figure 5

| 7.2.1 Calculate the bending moment at A, B, C and D as well as point “P” | (10) |
| --- | --- |

| 7.2.2 Calculate the shear force between AB, BC and CD as well as point “P” | (8) |
| --- | --- |

| 7.2.3 Draw the bending moment diagram. | (6) |
| --- | --- |

| 7.2.4 Draw the shear force diagram. | (6) |
| --- | --- |

| Use the following scales: |  |
| --- | --- |

| Space diagram 1:100 Bending moment diagram 1 mm = 10 kNm Shear force diagram 1 mm = 1 kN |  |
| --- | --- |
| TOTAL FOR SECTION B: | [40] [160] |

| TOTAL: | 200 |
| --- | --- |