GAUTENG DEPARTMENT OF EDUCATION SENIOR CERTIFICATE EXAMINATION WOODWORKING SG TIME: 3 hours MARKS: 200

| REQUIREMENTS: - A3-size Drawing paper INSTRUCTIONS: - Answer any FIVE questions. - Use both sides of the drawing paper. - Written answers must be done on the drawing paper. - Drawings are to be fully dimensioned and labelled in accordance with the SABS recommended practice for building drawings. |  |
| --- | --- |

| QUESTION 1 A front entrance door of 2 100mm x 960 mm x 45 mm has two raised panels and a high lock rail. The door is fitted in a frame with a semicircular fanlight. The fanlight is provided with two glass panes. The door and frame is finished with ovolo moulds. The frame is 115mm x 75mm, door stiles and top rail 115mm, lock and bottom rail 230mm and the transom is 115mm x 75mm. Draw the following: - To a scale of 1:10, a front elevation of the door, fanlight and frame - To a scale of 1:2, a horizontal section through the frame, door and portion of the panel | (30) (10) [40] |
| --- | --- |

| QUESTION 2 The components of a double casement window and frame have the following measurements: head of frame, stiles and mullion, 115mm x 75mm. Stiles of the sash 50mm x 45mm and the bottom rails 65mm x 45mm. Draw isometric views to a scale of 1:2 of the following: - The joint between the head of the frame and the mullion - The joint between the bottom rail and the stile of the sash. The components must be shown apart. | (20) (20) [40] |
| --- | --- |

| QUESTION 3 - Strip board panelling is fixed against a wall to a height of 1 200mm. Hardwood strips of 75mm x 22mm are used. The top of the panelling is finished off with a suitable capping which is 75mm wide. A skirting board and quadrant are used at the bottom. Draw to a scale of 1:10 a front elevation of the panel work approximately 1 500mm long with part of the rough ground strips shown uncovered. Also draw a vertical section through the panel work showing a method of fixing it to the wall. - Illustrate by means of a neat drawing the term secret nailing as applied to strip board panelling. | (34) (6) [40] |
| --- | --- |

| QUESTION 4 A South African type of roof truss has a pitch of 30o and is made of 115mm x 38mm timber. The roof is covered with corrugated iron sheeting, fixed to 75mm x 50mm purlins. The closed eaves project 450mm. The roof is supported on a 100mm x 38mm wall-plate and a 220mm brick wall plastered on the inside. Draw the following to a scale of 1:5: - A vertical section through a closed eave showing all details, including a portion of the corrugated iron sheeting, fascia board 230mm x 25mm, 115mm x 115mm gutter and a short length of the down pipe. 75mm x 38mm timber is used for the closed eaves. The drawing must also include a portion of the ceiling which consists of 6mm thick ceiling board fixed to 38mm x 38mm brandering, and finished off with a 125mm cove cornice. - A vertical section through the ridge of the roof. Show clearly the method of connecting the king post and rafters. The ridging and 75mm x 50mm purlins must also be shown. 4.3 An elevation of the bottom end of the king post. Show clearly the bottom end of the king post, struts and tie beam. | (22) (10) (8) [40] |
| --- | --- |

| QUESTION 5 An easy chair with shaped back and arm rests has the following inside dimensions: Height of back legs 965mm; between front legs 620mm; between back legs 576mm; between front and back legs 550mm; height of seat 425mm and the height of the arm rest 310mm above the seat. The front cabriole legs are made of material 75mm x 75mm. Draw to a scale of 1:10 a front and side elevation of the chair. | [40] |
| --- | --- |

| QUESTION 6 A straight flight of concrete stairs has seven steps, including a landing. The staircase starts on a concrete floor and ends with the landing against a one-brick (220mm) thick external wall. Use the specifications listed below and to a scale of 1:20 draw a vertical section through the length of the staircase showing the required formwork and necessary supports in position. Thickness of concrete slab : 100mm Landing : 900mm Riser : 150mm Tread : 280mm | [40] |
| --- | --- |

| QUESTION 7 A three-storey building with a floor to floor height of 3 000mm between storeys is to be supported by raking shores. - Draw to a scale of 1:50, a vertical section through the external wall showing the construction of the raking shores. - Name SIX safety precautions that you consider important when using a grinding wheel. TOTAL: | (28) (12) [40] 200 |
| --- | --- |