ME108	        1/7

DEPARTMENT OF MECHANICAL & MANUFACTURING ENGINEERING

MODULAR HONOURS DEGREE COURSE

LEVEL 1

SEMESTER 2

1997/98

EXPERIMENTAL METHODS

Examiners:	Mr J Lomax/Dr D Koshal

This is an Open Book Exam

Attempt FOUR questions only.					Time allowed: 2 hours

All questions carry equal marks			           Total number of questions = 5

The figures in brackets indicate the relative weightings of parts of a question.

At the conclusion of the exam you will be asked to leave your exam script plus

one log book for each subject (Dynamics, Materials, Manufacturing, Mechatronics

and Thermo Fluids)

Ensure that any log book loose papers are in the correct log book and that each

loose page has your name and subject recorded on it in the top right hand corner.

The following are supplied:

3 cycle Log/Log graph paper

Graph paper

ME108	2/7

1.	Experimental Errors - EXP12.

In the experiment a paper rule was made, calibrated and then used to measure a metal triangle.  The standard deviation in errors on subsequent measurements was taken as the standard deviation of the rule.  Statistical formulae were then used to determine the standard deviation of the periphery and the volume when measured with the paper rule.  Then the upper and lower limits based on one-standard deviation of the paper rule were compared with the values produced when a vernier calliper gauge was used to perform the measurements.

(a)	Manufacture and calibration

(i)	state how you made the paper rule

(ii)	show the calculation of the standard deviation of the rule 'rd'																(4)

(b)	Measurement of length of individual sides (A, B, C).

Use your results to complete a copy of the following table:

| Side | A | B | C |
| --- | --- | --- | --- |
| measured length of each side using rule '1' mm |  |  |  |
| upper limit of each side: (1 + rd) using rule mm |  |  |  |
| lower limit of each side: (1 - rd) using rule mm |  |  |  |
| measured length of each side using vernier mm |  |  |  |

(8)

THIS QUESTION CONTINUES ON THE NEXT PAGE

ME108	3/7

(c)	Determination of triangle periphery (sum of sides)

State the formula to determine the standard deviation of a sum.

Use your results to determine the following :

(i)	periphery based on rule measurements

(ii)	upper limit of periphery based on rule

(iii)	lower limit of periphery based on rule

(iv)	periphery based on vernier measurements					(6)

(d)	Overall

(i)	State the proportion of your actual vernier based results that 				were beyond the upper and lower limits of the results based 				on the rule

(ii)	discuss the proportion that you would expect to find beyond 				the limits

(iii)	discuss why these two proportions may be different				(7)

ME108	4/7

2.	Materials Engineering - MAT 13

For this question use the data you gained on measuring properties of EN1(T), EN1(Q), EN16(T), EN16(Q) (where (T) means tempered or air cooled, and (Q) means quenched in oil)

(a)	Enter your data into a copy of the following table allowing for rig 		compliance:

| Material | EN1(T) | EN1(Q) | EN16(T) | EN16(Q) |
| --- | --- | --- | --- | --- |
| Elastic modulus (GPa) |  |  |  |  |
| Yield tensile stress (MPa) |  |  |  |  |
| Ultimate (or break) stress (MPa) |  |  |  |  |
| % Elongation at break |  |  |  |  |
| % Reduction in area at break |  |  |  |  |
| Vickers Hardness number |  |  |  |  |
| Rockwell B hardness |  |  |  |  |
| Rockwell C hardness |  |  |  |  |

(8)

(b)	(i)	Produce a plot of the various hardness results versus yield

tensile stress.									(8)

(c)	Explain the following :

(i)	the reasons why the curves for the plot based on yield stress

are neither linear nor similar to each other

(ii)	which hardness scale provides the best predictor of tensile 					stress  and explain why?

(iii)	why necking occurs in some specimens and not in others?

Explain why necking does not necessarily start immediately.		(9)

ME108	5/7

3.	Thermofluids - THF 11

(a)	Define the Reynolds number								(5)

(b)	Describe how this number is used in the analysis of laminar and 			turbulent flows										(5)

(c)	Write the Bernoulli equation taking into account the energy head			 			loss along the pipe and explain the meaning of the terms in this			 			equation										(5)

(d)	Describe the concept of the non-dimensional friction factor, its						contribution to the energy head loss and its dependence on the 						Reynolds number									(10)

ME108	6/7

4.	Electrical Engineering - ELE 11

(a)	Describe the fundamental purpose of a capacitor.					(3)

(b)	Assuming that the 100 mF capacitor shown in fig Q4 is initially 				uncharged and then the switch is closed:

(i)	Sketch the transient voltage response across the capacitor to		 			scale on graph paper

(ii)	Determine the time constant from this graph and the 					voltage at that point.

(iii)	Calculate the theoretical time constant from the component 				values

(iv)	Discuss the reasons for possible differences between the

theoretical experimental time constants					(12)

(c)	Sketch the likely transient response if the resistor were halved in			 			value, and calculate the value of the time constant					(3)

(d)	Draw a circuit for a low pass filter using a 6.3 mF capacitor and a 22 kΩ 					resistor.  Calculate the 'characteristic' frequency (or bandwidth) and 					sketch the frequency response on a log scale						(7)

sw	R

10v	10k

100mF

Figure Q4

ME108	7/7

5.	Manufacturing Engineering - MAN 12 and MAN 13 with referencce to

figure Q1 in MAN12 and the associated sample program.					(5)

(a)	State the meaning of the five individual code words in block 5:

G99 G40 G96 S150 M13

(b)	State the function of the five individual blocks comprising the part-						off module										(10)

(c)	Rewrite the 10 block module defining the roughing cycle from

G71 to G40 to produce the different geometry shown in Figure Q5 below.

Assume that the billet and tools remain the same.					(10)

30

26

19

16

11

5

6f

6R

18f

3R

Part off	24f

Figure Q5