ME 202    1/5

UNIVERSITY OF BRIGHTON

DEPARTMENT OF MECHANICAL & MANUFACTURING ENGINEERING

MODULAR HONOURS DEGREE COURSE

LEVEL 2

SEMESTER 2

1997/98

MATERIALS ENGINEERING

CHIEF EXAMINER: DR J RHODES/DR M PHILIP

Attempt FIVE questions only.					           Time allowed: 3 hours.

Number of questions = 8.

No more than THREE from either

section A or section B.

All questions carry equal marks.

The figures in the right hand margin indicate the relative weighting of parts of a question.

The following is provided:

Graph paper.

ME 202      2/5

SECTION A

1.	A vertical force P is applied at a point A of the machine component		 	shown, in figure Q1.  If the yield strength of the material from which		 	the component is made is 200 MPa, what, according to the "Maximum		 	Shear Strain Energy Criterion", would be the value of P beyond which		 	failure would occur at point H?

2.	For the beam shown fig Q2 determine the reactions at A and B and the		 	end moments caused by the ends being encastre'.

ME 202      3/5

3.	For the uniform rod with circular form radius 2m and anguler length  radians		 	shown in figure Q3 and using Castigliano's theorem determine the horizontal		 	deflection caused by a vertical load of 10kN.  Take EI to be 400 MNm2.

Figure Q3.					(20)

4.	A closed cylinder of external diameter 400 mm and internal diameter 300 mm

is subject to an external pressure of 40 MPa and the condition that the increase

in external diameter caused by the internal pressure P should not exceed 100 mm.		  	Given that E and n for the material are 200 GPa and 0.29 respectively, determine

the value of P.										(20)

ME 202     4/5

SECTION B

5.	Briefly discuss three of the following topics in the context of polymeric 	materials:-

(a)	Condensation polymerisation and its products

(b)	Molecular weight distribution and its assessment

(c)	Stereoregularity and its effect on the crystallisation and

mechanical properties of thermoplastics

(d)	The glass transition temperature in thermoplastics.

(20)

6.	Syndiotactic polypropylene is a semicrystalline polymer with the 			following properties:-

Density = 905 kg/m3

Glass transition temperature, TG = - 10 oC

Maximum operating temperature, TOp = 105 oC

Physical properties at T = 21 oC :-

Tensile strength = 33 MN/m2

Flexural modulus = 1.5 GN/m2

Elongation at break = 150 %

Sketch and describe the load / extension characteristics of such a polymer		 	at T = 21 oC when subjected to a tensile test at a relatively low strain-rate.

(12)

How is this behaviour changed if the test is repeated at a higher  strain-			rate while keeping the temperature constant?			         		        (4)

State two examples of products manufactured from polypropylene,		 	giving reasons for the choice of material.					         (4)

ME 202   5/5

7.   	(a)	Discuss the advantages and disadvantages of the following when		 		used in fibre form in fibre reinforced composites giving one		 			example in each case:-

(i)	E-glass and S-Glass

(ii)	Carbon

(iii)	Kevlar							      		(12)

(b) 	The thermoplastic polymide (Nylon) 6,6 is to be reinforced with		 		40% by volume of unidirectional carbon fibres in the manufacture		 		of a component. The properties of the individual materials are 					given below:-

Material	Density	  	 Tensile Strength		Young's Modulus

(kg/m3)	(MN/m2)	(GN/m2)

Nylon 6,6	   1400	35	2.5

Carbon fibre	1800	2100	400.0

Calculate the Young's modulus and strength of the composite along the			 	fibre direction and the fraction of the applied force which will be taken by the 			fibres.									       		 (8)

8.	(a)	Describe the essential features of the standard linear solid model			 		when used to describe the behaviour of linear viscoelastic			 			materials indicating its limitations.				    		  (14)

(b)	A viscoelastic polymer obeying the Boltzmann superposition		 			principle is subjected to the following load cycle. At time, t = 0, a		 		tensile stress of 10 MNm-2 is applied and maintained for 100 s.		 			The stress is then removed instantaneously.  If the creep	 				compliance is given by,

J (t) = Jo [ 1 - e-t/t]

where  Jo = 2 m2GN-1  and  = 200 s, what is the net creep strain	 			after 200 s?.								  	    (6)