ME203            1/10

DEPARTMENT OF MECHANICAL & MANUFACTURING ENGINEERING

MODULAR HONOURS DEGREE COURSE

LEVEL 2

SEMESTER 1

1997/98

MECHATRONICS

Examiners:	Mr J Lomax/Dr D F Pearce

Attempt FIVE questions only					Time allowed: 3 hours

All questions carry equal marks			         Total number of questions = 8

The figures in brackets indicate the relative weightings

of parts of a question.

The following charts, tables and other special requirements

are supplied:

3/4 cycle log/log graph paper

NOTE

attached is:

- Annex A - data on electrical components

- Annex B - summary of Quick Basic Syntax

ME203         2/10

1.	An engineer plans to monitor the axial force (F) in a tensile support rod by	 	installing two gauges to sense axial strain on either side of the rod,

figure Q1a.  He then plans to connect these through a Wheatstone Bridge		 	to a differentially configured Op-amp figure Q1b.

DATA

Rod diameter			D	8 mm

Rod elastic modulus		E	90 GPa

Strain Gauge factor		k	2.05

Strain gauge resistance		Rg	100 Ω

Resistor			R1	100 Ω

Resistor			R3	5 kΩ

Resistor			R4	100 kΩ

(a)	Calculate the value of R2 to ensure that the voltage across the bridge			 			(Vb) is 2 V										(10)

(b)	Draw a block diagram that links the force (F) with the op-amp			 				output voltage (Vo) showing the transfer function of each block in

terms of the variables D, E, k, Rg, R1, R3, R4, Vb.					(5)

(c)	Evaluate the overall transfer function dVo/dF using the values			 				given in the data table in Annex A.							(5)

Figure Q1a							Figure Q1b

ME203         3/10

2.	(a)	Describe the relative advantages of using a Programmable Logic		 				Controller (PLC) versus a personal computer (PC) for monitoring			 			discrete switching sensors.								(5)

(b)	A security system has sensors for detecting fire, intruders, door and 				windows opening defined by the symbols f, i, d and w respectively. 		 		The alarm A will be activated when the following condition is met

(i)	Reduce the expression for (A) using Karnaugh or other			 					methods to two alternate simplified expressions					(5)

(ii)	Draw a circuit which uses a PLC and relay to switch a

500 Watt, 220 V ac motor (see annex A)					(5)

(iii)	Draw a ladder diagram to implement the un-simplified 								condition in (b) above								(5)

ME203         4/10

3.	(a)	Ac induction motors and universal ac/dc commutator type motors			 			are both used widely in houses, offices and factories.  Describe two			 			advantages that each motor has and name an example where these			 			advantages are exploited.								(4)

(b)	A dc permanent magnet motor featuring a commutator operates at		 		12 V with a maximum power rating of 25 W and a maximum no-				load running speed of 8000 rev/min.  Assume a linear relationship		 		between torque D speed.

(i)	Determine the speed and torque at maximum power				(2)

(ii)	Draw to scale a graph of speed versus torque for the motor			(2)

(iii)	Determine the torque and current at stall					(2)

(c)	(i)	Draw a circuit comprising an op-amp and a power transistor			 				(see annex A) to control the speed of this motor in response			 				to an input voltage at the op-amp over the range 0-to-12 V			 				(ignore any 0.7 V junction drop)						(4)

(ii)	Determine the current gain needed by the power transistor at			 				stall										(2)

(iii)	Identify the appropriate power transistor (see Annex A)			(2)

(iv)	State the maximum power that would be dissipated by the			 				transistor									(2)

ME203         5/10

4.	In an accelerated life test on 100 universal couplings the number		 	remaining 'n' after a period of time 't' are recorded below.  The test		 	engineer wants to see whether the data fits an expression of the form

n = c.td

Time t (h)	1	3	10	30	100	300	1000

Number n	100	65	40	25	15	10	5

(a)	(i)	Draw a graph to scale of loge(n) versus loge(t)					(5)

(ii)	Determine the values of 'c' and 'd'						(5)

(b)	Write a program in Quick Basic employing structured programming			 			principles that accepts data of 'n' and 't', calculates loge(n) and			 				loge(t) and prints out using linear regression the values of 'c' 							and 'd'.  The equations for linear regression are given below.  Annex			 			B provides a summary of Quick Basic syntax.						(10)

The following equations may be used

Average of data points		xav=Sxin		yav=Syin

Gradient of line of best fit	a1=n.Sxi.yi−Sxi.Syin.Sxi2−Sxi2

Intercept with the vertical axis	ao=yav−ai.xav

ME203        6/10

5.	An engineer working in a steel strip rolling mill wants to use a strain 			gauge to detect the strain in the frame of one of the rolling machines and 			then transmit the signal back to the control room 100 meters away at the	 	other end of the mill.  The intent is to use a bridge circuit consisting of			 	one or more op-amps and a length of multi-core cable to connect the two 		locations.

(a)	(i)	Should the cable be twisted and if so why?					(2)

(ii)	At which end should the first op-amp be and why?				(2)

(iii)	Should the op-amp be configured as a voltage or current 							driver and why?								(2)

(iv)	Draw an op-amp based current driver circuit					(2)

(b)	(i)	Draw a graph of a first order low pass filter based on a dc gain			 				of 10:1 and a cut off-frequency of 5Hz						(3)

(ii)	What is the gain at 50Hz							(3)

(c)	(i)	Design a circuit using an op-amp to provide the filtering 							requirements specified above							(3)

(ii)	If the input resistance is 10 kΩ calculate the value of the		 					components in your circuit.							(3)

ME203        7/10

6.	The program below was one of ten programs used in the laboratory		 	sessions supporting this module.  This program inputs an analogue signal		 	and displays the wave-form graphically using the full dimensional		 	capability of the monitor.

(a)	State briefly the function of each of its six modules marked A ...F			(6)

(b)	What is the function of each of the following program lines:

(i)	200

(ii)	340

(iii)	440

(iv)	530										(4)

(c)	Carefully rewrite the program to plot the same signal in the top left			 			quadrant (quarter) of the monitor's screen at half the previous			 				vertical and horizontal resolution							(10)

QUESTION 6 CONTINUES ON THE NEXT PAGE

ME203       8/10

100 REM INPLOTF.BAS'  **Module A**

110 REM A-to-D prog latest version as 3.8.97

200 DIM y(1000)'  **Module B**

210 INPUT "Key in max voltage pk-to-pk (v) "; vmax

220 PRINT "Keyin time scale factor 1 ...50 and adjust "

230 INPUT " To get 3 to 4 cycles along marked baseline ";j1

240 OUT &H703, &H92

250 k = 0: j0 = 1

300 SCREEN 1'  **Module C**

310 COLOR 1, 1

320 CLS

330 LINE (50, 50)-(50, 150)

340 LINE (50, 150)-(250, 150)

400 start = TIMER'  **Module D**

410 FOR n = 1 TO 1000

420 OUT &H702, 3

430 OUT &H702, 2

440 y(n) = ((INP(&H701) AND &HF)* 16) + (INP(&H700) AND &HF0) / 16

450 NEXT n

500 trecmax = TIMER - start'  **Module E**

510 FOR n = 1 TO 1000

520 t = n* trecmax / 1000

530 tplot = 50 + t * 200 * (j1) / trecmax

540 v = y(n) * 10 /255

550 vplot = 150 - v * (100 / vmax) * j0

560 PSET (tplot, vplot)

570 NEXT n

600 PRINT "tmin="; 0; " tbaseline (s) ="; trecmax / (j1)'  **Module F**

610 PRINT "vmin=0"; 0; " vmaxline (v) ="; vmax / j0

620 END

ME203       9/10

7.	With regard to computer wide-area-networks

(a)	State the function of each of the following protocols:

telnet, ftp, http									(3)

(b)	State the full name of three of the following acronyms which are 				parts of addresses used widely in the UK academic community:

niss, hensa, janet, bids								(3)

(c)	Describe the difference between Search Engines and Subject 							Catalogues on the World Wide Web.							(3)

(d)	State two methods of accessing computers at the University of		 			Brighton (UoB) from each of the two following remote situations.

(i)	A computer with a modem in a domestic environment.				(2)

(ii)	A computer with Internet or Janet access in another 						                        University.									(2)

(e)	What are the claimed advantages of the new ATM network planned			 			for the UoB compared to the existing network used to link major			 			buildings and remote UoB campuses.							(3)

(f)	Describe the proposed network using a block diagram if possible.			(4)

ME203       10/10

8.	The management team of a chemical company intend to design a new		 	asymmetric combustion chamber for generating heat from waste material, 		figure Q8.	 The team is considering the possible application of a Computer		 	Fluid Dynamics (CFD) package to this problem.

Consider that you are asked to provide the following advice to the team:

(a)	What are the main advantages of using a CFD package					(5)

(b)	What are the main limitations of using CFD						(5)

(c)	What are the governing equations which are solved in CFD				(5)

(d)	Which of the following types of grid would be suitable for tackling		 		this problem and why:

-	Cartesian

-	Cylindrical

-	Body Fitted Co-ordinates (BFC)						(5)

Figure Q8