- SCHOOL OF ENGINEERING

MODULAR HONOURS DEGREE COURSE

LEVEL 2

SEMESTER 2

1998/99

MANUFACTURING ENGINEERING

Examiners:  Dr D Koshal / Mr H L Cather

Attempt FIVE questions only	Time allowed: 3 hours

Total number of questions = 8

All questions carry equal marks.

The figures in bracket indicate the relative weighting of parts of a question.

Special requirements: 	None

1. 	(a)	With the aid of sketches briefly explain the difference between direct

extrusion and indirect extrusion.	(6)							(6)

(b)	In the hot extrusion process briefly discuss the effect of velocity upon the total ram pressure and the process accuracy.	(4)						(4)

(c)	Two billets both of diameter 50 mm and lengths 160 mm and 210 mm respectively, are extruded under identical conditions. The maximum pressure required in each case was measured as 420 MPa and 450 MPa respectively. Calculate the coefficient of friction that exists between the billet and container.	(10)						(10)

2. 	(a)	Briefly explain what is meant by the term ‘Process Capability’.	(2)			(3)

(b)	A machine process that was believed to be subjected only to ‘common causes’ was monitored with ten samples, each of five components taken every twenty minutes. Details of the samples are given in Table Q2a

| Sample |  |  | Dimension |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 8.04 | 8.01 | 7.98 | 8.02 | 8.00 |
| 2 | 8.02 | 7.97 | 7.96 | 8.01 | 8.02 |
| 3 | 8.01 | 8.07 | 7.99 | 8.03 | 8.00 |
| 4 | 7.98 | 7.97 | 8.02 | 7.98 | 7.98 |
| 5 | 7.99 | 8.03 | 7.98 | 8.02 | 8.01 |
| 6 | 8.02 | 7.95 | 8.04 | 8.02 | 7.95 |
| 7 | 8.00 | 7.99 | 8.01 | 8.02 | 8.01 |
| 8 | 7.99 | 8.02 | 8.00 | 8.04 | 8.09 |
| 9 | 8.03 | 8.04 | 7.99 | 8.02 | 7.94 |
| 10 | 8.02 | 7.98 | 8.00 | 7.99 | 8.02 |

Table Q2a

QUESTION 2 CONTINUES ON PAGE 3/8

(i)  	Using the appropriate constants from Table Q2b calculate the control

limits on means  and ranges.  Plot the samples on the control charts for

Mean and Range.	(14)			(13)

(ii)      Comment on the drift and variability of the process.	(4)

|  | Normal mean chart |  | Range chart |  |
| --- | --- | --- | --- | --- |
| Sample size | Warning limit | Action Limit | Warning limit | Action limit |
| n | A' 0.025 | A' 0.001 | D' 0.975 | D' 0.999 |
| 2 | 1.128 | 1.937 | 2.810 | 4.120 |
| 3 | 0.668 | 1.054 | 2.170 | 2.980 |
| 4 | 0.476 | 0.750 | 1.930 | 2.570 |
| 5 | 0.377 | 0.594 | 1.810 | 2.340 |
| 6 | 0.316 | 0.498 | 1.720 | 2.210 |
| 7 | 0.274 | 0.432 | 1.660 | 2.110 |
| 8 | 0.244 | 0.384 | 1.620 | 2.040 |
| 9 | 0.220 | 0.347 | 1.580 | 1.990 |
| 10 | 0.202 | 0.317 | 1.560 | 1.930 |
| 11 | 0.186 | 0.294 | 1.530 | 1.910 |
| 12 | 0.174 | 0.274 | 1.510 | 1.870 |

Table Q2b   Constants for Limits on Mean and Control Charts

3.	(a)	With the aid of sketches show how the force varies with the following

parameters in orthogonal cutting.

- Surface speed	(2)

- Feed rate	(2)

- Depth of cut.	(2)

(b)	In a turning test carried out on mild steel the following data were obtained using orthogonal cutting conditions.

Tool rake angle	13o

Cutting speed	50 m/min

Depth of cut 	0.15 mm

Width of cut	3.5 mm

Tool contact length	1.0 mm

Tangential force 	460 N

Vertical force	210 N

Chip thickness	0.45 mm

Using Merchant’s analysis determine by graphical means or calculations,

(i)  	the angle of  inclination of the plane of shear	(7)				(8)

(ii)	the coefficient of friction between the tool and the chip.	(7)		(8)

4.	(a)	Discuss the SIX common modes of cutting tool failures and explain how, in

each case, it can be reduced or eliminated.	(15)							(15)

(b)	Briefly discuss the limitations of Taylor’s cutting tool life equation.	(5)

5.	The sliding dog shown in figure Q5 is machined from 50 mm diameter steel bar in batches of 200. Devise a suitable machining sequence to produce the dog on a programmed capstan lathe and a horizontal milling machine.

Calculate the minimum floor to floor time using the following machining information:-

# Milling

Cutter data

Number of teeth        	10

Diameter	100 mm

Width	14 mm

Maximum speed	0.7 m/s

Feed per tooth	0.1 mm

Loading and unloading time	60 s

Fixture and indexing time	20 s

# Turning

Loading and unloading

time (2nd operation)	30 s

Maximum turning speeds

Carbide tools 	0.5 m/s

HSS cutters and drills 	1 m/s

Reamers 	0.4 m/s

Maximum feeds

Turning and drilling	0.2 mm/rev

Reaming 	0.5 mm/rev

Parting off and recessing 	0.05 mm/rev

Turret indexing and bar

feeding time	3 s

Assume that the maximum and minimum spindle speeds on the lathe are 2500 and 60 rev/min respectively, and that within this range all the calculated maximum speeds are available.			(20)

- (a)	Briefly discussed the possible potential benefits which can be realised by

designing electronic circuits using surface mount components rather than

conventional components. 		(10)

- Discuss the significant changes which have to be made in the printed circuit

board when switching from standard insertion mount components to the new high density surface mount components. 	(10)

- A mild steel component, manufactured in a large batches, is to be reduced from

60 mm to 40 mm outside diameter on a lathe using carbide cutting tools having a recommended cutting tool life of 25 minutes.

The following feeds and speeds are available on the lathe:

Feed (mm/rev)

0.04  to 0.4 in steps of  0.02

Speed  (rev/min)

50,   70,   98,   137,   192,   269,   377,   527,  738,   1033,   1446,   2025

The maximum input to the drive motor is 15 kW and the transmission of efficiency of the maximum power is a 65%

(a)	Calculate the best value of cutting speed and feed if

V  T0.25  f0.35   =   6.4

and 	P = 1600  f0.87  d  V

where 	 P = 		Power (Watts)

T =  	Tool life (minutes)

V  = 		Cutting speed (m/s)

f   = 		Feed rate (mm/rev)

d  =    	Depth of cut (MM)									depth of cut (mm)

If the tangential cutting force is limited to 2500 N select the nearest feed (ie above or

below the calculated value) and select the nearest spindle speed below those giving

optimal conditions. 	(16)

(b)	Calculate the metal removal rate for the cutting condition.	(4)

8.	(a)  	With the aid of sketches identify all the forces in orthogonal cutting and

oblique cutting operations. Which force contributes most to the power

required during cutting?	(5)

- (i)	Discuss why studying the types of chips produced is important in

understanding cutting operations. 	(2)

- Discuss the differences between continuous chip and discontinuous

chip and the conditions which give rise to each.	(13)