ME207   1/5

- SCHOOL OF ENGINEERING

- MODULAR HONOURS DEGREE COURSE

- LEVEL 2

- SEMESTER 1

1998/99

- THERMOFLUIDS

- EXAMINERS : Dr F McCluskey/Dr S Sazhin

___________________________________________________________________________

Attempt FIVE questions only.	Time allowed: 3 hours

All questions carry equal marks.	Total number of questions = 8

- The figures in brackets indicate the relative weightings of parts of a question.

The following are provided:

Thermodynamic & Transport Properties of Fluids - SI Units

Pressure - Enthalpy Chart for R12

Pressure - Enthalpy Chart for R134a

Psychiometric Chart for Air

ME207   2/5

1.	(a)	Sketch the graph of the drag coefficient CD for a sphere as a function

of Reynolds number.	(3)

- Define the terms "drag coefficient" and "Reynolds number" and write down their mathematical expressions.	(6)

- If CD =  when Re < 0.2, derive from the definitions the expression for the drag force on a sphere:

- D = 3  U  d

where all terms have their usual meaning.	(5)

- Dust particles have a density of 2000 kg/m3 and a mean diameter of 30m.  Determine the terminal velocity of the dust particles in still air.  (Neglect buoyancy)

Air dynamic viscosity  = 1.5 x 10-5 Pa.s, density  = 1.3 kg/m3.  Assume the Stokes flow approximation and check this assumption from your solution.	(6)

ME207   3/5

2.	(a)	Sketch the air flow pattern behind an aerofoil of finite length and

discuss why such a pattern arises.	(6)

- How are lift and drag forces effected by this three dimensionality and name all the components of the drag force in this case.	(5)

- Calculate the component of drag on a bi-plane due to the presence of six struts between the wings in the following circumstances:

strut diameter:	1 cm

strut length	1 m

plane velocity	80 km/h

air density :	1.2 kg/m3

air viscosity :	1.7 10-5 N s m-2

take the value of CD at the appropriate Reynolds number to be 1.1.	(9)

3.	(a)	Derive expressions for the volumetric and torque efficiencies of a

pump.  Define all terms used.	(8)

- Define the overall pump efficiency and show how it is related to the efficiencies defined in part (a).	(4)

- What is the overall efficiency of a pump with a delivery of 15 l/min, a displacement of 10 cm3/rev, a pump speed of 2000 rev/min, a torque of 29.8 Nm at the pump shaft, operating against a maximum pressure rise of 100 bar?	(8)

ME207   4/5

4.	(a)	Explain the terms:

- pressure drag

- surface drag

- total drag

- deformation drag	(8)

- What is the point of streamlining fast moving subsonic objects?	(2)

- How does streamlining effect

- pressure drag

- surface drag?	(4)

- Is drag increased or decreased when the flow becomes turbulent?	(3)

- How is it possible to induce turbulent flow around a streamlined object?	(3)

5.	(a)	A refrigerator using refrigerant 12 and operating the ideal cycle has

evaporator and condenser pressures of 2 bar and 8 bar respectively.

The temperature at compressor outlet is 60 C.  Calculate the  coefficient of performance.	(8)

- The R 12 is replaced by R134a.  Volumetric flowrate is adjusted such that the pressures are not changed, nor are the evaporator and condenser exit temperatures.  Calculate the COP.	(12)

ME207   5/5

- Water at 65C flows through a 5 cm bore steel pipe 1 cm thick.  The surroundings are at 20C.  The pipe conductivity is 52 W/m K and the surface heat transfer coefficients inside and outside are 1136 W/m2 K and 13 W/m2 K respectively.  Calculate the heat loss per metre length of pipe and the surface temperatures.	(20)

7.	(a)	The temperature of the windows in a house on a day in winter is 5 C.

When the temperature inside the room is 23 C and the barometric

pressure is 74.88 cm Hg, what would be the maximum relative

humidity that could be maintained in the room without condensation

on the window panes?	(5)

- Under these conditions find:

- the partial pressures of the water vapour and air	(5)

- the specific humidity of the mixture	(5)

- the density of the mixture.	(5)

8.	A closed system undergoes a reversible process in which the temperature increases from 222 K to 444 K.  The heat and work transfers per degree temperature rise are given respectively by dQdT= 450 J/K and

dWdT = 125 J/K.

- Calculate the change in

- the internal energy	(10)

- entropy for the process	(10)