ME 302	        1/7

DEPARTMENT OF MECHANICAL & MANUFACTURING ENGINEERING

MODULAR HONOURS DEGREE COURSE

LEVEL 3

SEMESTER 1

1997/98

DYNAMICS AND CONTROL

Examiners:	Dr Dave Pearce/Dr Carl Hogg

Attempt FOUR questions only.					Time allowed: 3 hours

Total number of questions = 6

All questions carry equal marks.

The figures in brackets indicate the relative weightings

of parts of a question.

The following charts, tables and other special requirements

are supplied:

Graph paper

Mathematical Formulae (B&C)

ME 302         2/7

1.	A unity feedback closed-loop position control system operates with an 			oscillatory performance such that  = 0.5 and a settling time of 1.33 			seconds.

It is proposed to use a lead/lag network in the forward path to improve 				the time response to  = 0.707 and settling time of 0.8 seconds.

If the original forward path transfer function is F(s) = Ks2+6s+18 and using		  the root-locus technique.

Determine:

(a)	the required compensating lead/lag network to satisfy the desired			 	closed-loop response									(12)

(b)	a root-locus diagram for the control system						(8)

(c)	the expected change to steady state performance.					(5)

ME 302         3/7

2.	(a)	A transducer mechanism designed to measure angular velocity is			 	shown in figure Q2.  The input, u, to the transducer is an angular			 	velocity (rad s-1) and the output, y, is an angular displacement (rad).

Using the state variables x1 = y and x2 = y· draw the state variable 				diagram and formulate the state and output equations.					(10)

(b)	Assuming the transducer starts from rest and using the data

provided below, calculate a time simulation of the output, y, in steps

of 50 ms for a period of 250 ms when the input, u, experiences

a step input of 1 rad s-1.								(15)

Data

k = 3.2 Nm rad-1

C = 0.8 Nms rad-1

I = 0.2 kg m2

Figure Q2

ME 302         4/7

3.	A robot arm used in a car assembly plant is shown in figure Q3.  The gripper has three degrees of freedom; two angular, q and a about axes XX and YY respectively and one linear in the radial direction, r, along the arm.  The gripper and component are to be considered as a concentrated mass, m, acting at the instantaneous radial distance r.

Determine:

(a)	a general equation for the kinetic energy of the component			`	(10)

(b)	the general expressions for the torques about the axes XX and YY

due to the movement of the component							(15)

Lagrange general force equation:

ddt¶T¶q·i−¶T¶qi=Qi

where  T is the kinetic energy

qi is a generalised co-ordinate

Qi is a generalised force

Figure Q3

ME 302         5/7

4.	A machine tool when operating at constant speed generates a force

P1 = 100 sin 70t acting in the vertical direction.  To reduce the effect from		  this force acting on the factory floor the machine has been mounted on four 	        isolation pads, shown in figure Q4.  It was then subsequently found that the		 machine tool structure experienced an unsatisfactory vibration in the vertical direction.

(a)	Using the data provided below design an auxiliary mass and spring 				system for the machine to minimise the vibration amplitude.				(10)

(b)	Determine the magnitude of the force applied to the factory floor	 	from;

(i)	using only the vibration isolation pads, and

(ii)	using the isolation pads and the auxiliary mass and spring 						system.										(15)

Data

Mass of machine tool = 1000 kg

Isolation pad stiffness = 5 x 105 N m-1

Isolation pad damping = 20 x 103 Ns m-1

Auxiliary mass = 20 kg

Figure Q4

ME 302         6/7

5.	The design of a railway wagon bogie showing the principal masses and

springs is illustrated in figure Q5.  Using the data provided below,

determine all the natural frequencies of vibration in the horizontal

direction and verify the modal shapes.								(25)

Mode 1			   Mode 2			      Mode 3

x1x2			x1x2			x1x2

Data

Mass m1 = m3 = 100 kg

Mass           m2 = 50 kg

Stiffness k1 = k3 = 10 x 104 N m-1

Stiffness          k2 = 5 x 104 N m-1

Figure Q5

ME 302         7/7

6.	The hydraulic actuator for positioning a flight simulation platform is shown		    in figure Q6, where the position error supplies oil to the cylinder via a spool-valve. 	     It is to be assumed that the equivalent unity feedback control system has an open-loop transfer function F(s) = 20s3+13s2+32s and the closed-loop state equations are:

x·1x·2=0      1       00      0       1

(a)	Discuss how state feedback signals might be obtained from this

system and used to improve the dynamic performance of the

output.											(8)

(b)	Using the state equations for the system determine the matrix of 		state feedback coefficients necessary to generate closed-loop poles at

-8 and -6±j2.										(17)

Figure Q6