ME 302   1/7

- SCHOOL OF ENGINEERING

- MODULAR HONOURS DEGREE COURSE

- LEVEL 3

- SEMESTER 1

1998/99

- DYNAMICS & CONTROL

- CHIEF EXAMINER : Dr D Pearce/Dr J Leary

_____________________________________________________________________________

Attempt  FOUR questions only	Time allowed : 3 hours

Total number of questions = 6

- All questions carry equal marks

- The figures in brackets indicate the relative weightings

of parts of a question.

- The following special requirements are supplied

Graph paper

Mathematical Formulae (B & C)

ME 302   2/7

- An industrial process control system which features a first order lag is illustrated in Figure Q1.  In order to reduce steady state errors a lag/lead compensating element has been introduced in the forward path of the control loop.  Both the coefficients K and T are variable and it is desirable to determine their influence

on the output response.

- With T = 0, draw a root-locus for the combined system and describe the  
closed-loop performance if K = 4.							(10)

- Using the s-plane analysis draw a root-contour with T variable and
K = 4.											(10)

c)	Discuss the influence the coefficient T has on the time response of the               				system.										(5)

Figure Q1

- ME 302	3/7

- The schematic  diagram shown in Figure Q2 represents a rig for testing a 
structural shock absorber.

a)	Draw the state variable diagram and use this to determine the state

and output equations.      							(12)

- If the system is initially at rest and the input is subjected to a ramp 
signal Ut=0.5t, calculate a time simulation of the output in steps of 
50 ms for  period of 250 ms.		     					(13)

- DATA

K=500 N m−1, C=100 N s  m-1, m = 25 kg

- Figure Q2

ME 302	4/7

3.	A boring bar and its equivalent system, shown in Figure Q3, are to offer maximum 	resistance to transverse vibration.  The boring bar is represented by an equivalent
          	mass and spring acting at the cutting tool.   The vibration absorption is achieved with 
 	the auxiliary mass, spring and damper system located inside the boring bar.

If the rotational velocity of the bar is 600 rev/min determine :

a)	the impedance matrix for the equivalent system;				     (6)

b)	the necessary auxiliary spring stiffness;					     (7)

c)	the boring bar vibration amplitude resulting from a transverse
		cutting force variation P1 = 10 Sin 20  t.	    				   (12)

- DATA

Equivalent mass  of the boring bar			5 kg

Equivalent transverse stiffness of the boring bar	32 x 104 N m-1

Auxiliary mass						0.5 kg

Auxiliary damping						32 N s  m-1

- Figure Q3

ME 302                5/7

4.	A position control system, shown in Figure Q4, has been designed using a hydraulic
 actuator and spool valve.  The input to the valve is a mechanical movement which is proportional to the position error.  The equivalent unity feedback system has a 
forward path transfer function                                                                                  	 						                                								Fs=K1+s2+s10+s

a)	Draw a root-locus diagram for this system and estimate the required value

for the gain K to give the closed loop time response a damping ratio

= 0.707 										   (10)

b)	To maintain  = 0.707 it is required to increase the speed of response 
            such that the actuator settling time is 1.6 s.  Design a suitable spring and

dashpot compensator that can be incorporated within the hydraulic

spool-valve.	     									   (15)

Figure Q4

ME 302 	6/7

- An electro-mechanical positioning system operates in closed-loop as shown in

figure Q5.  It may be assumed that the system has three states which are

position x, velocity x and electrical current I.

Determine for the closed-loop system:

a)	The state and output equations for the system shown in Figure Q5.			     (8)

b)	A matrix of state feedback coefficients which will ensure the system

has poles at – 4, – 5±j5								   (12)

c)	A sketch of the output response, when the state feedback is included, for

a step input command.								     (5)

Figure Q5

ME 302	7/7

6.	Part of a paddle mechanism used for mixing chemicals in the process industry

is shown in figure Q6.  Assume that the paddle blade is constructed using a
 	rectangular sheet of material of constant thickness and with a mass of 100 kg.

a)	Explain why the paddle blade is out of balance on its axis of rotation. 	(8)

b)	Using the Euler equations determine the out of balance couple, in both

magnitude and direction, acting at the bearings when the rotation                  			            velocity of the paddle is 60 rev/min.						(17)

Figure Q6.