ME304   1/6

SCHOOL OF ENGINEERING

## MODULAR HONOURS DEGREE COURSE

LEVEL 3

SEMESTER 1

1998/99

FLUID DYNAMICS

Examiners:  Dr John Kearsey/Dr Sergei Sazhin

Attempt FIVE questions only					Time allowed:  3 hours

Total number of questions = 8

All questions carry equal marks.

# The figures in brackets indicate the relative weightings

# of parts of a question.

The following charts, tables and other special requirements

are supplied:

Prandtl Meyer Expansion Tables

Tables for Plane Oblique Shocks

Data Sheets

ME304   2/6

- (a)		If  is the semicone angle of a Mach cone, show that M = 1/sin 

and explain how  would be effected by altitude. M is the Mach

number.  Assume the velocity V of the relative flow is constant.			(6)

- Describe, using clearly labeled diagrams, how shock waves formed

on a body moving at Mach one, differ from those formed around a

body moving at supersonic speed.							(8)

- (i)	Explain what is meant by an expansion fan created by a

supersonic flow around a sharp 5 convex corner.  Indicate

the changes you would expect in direction, Mach number,

static pressure and temperature.

- Show how this case differs from a supersonic flow around a 5

radius concave corner.							(6)

- (a)		Derive from first principles, stating all assumptions used, that:

ToT=1+γ−12M2

where	To is the total temperature

T is the static temperature

M is the Mach number

 is the ratio of specific heats.							(12)

- An aircraft is flying at 183 m s-1 at an altitude where the ambient 				temperature is 20 C and the static pressure is 0.816 bar.  Calculate

the stagnation pressure at the leading edge of the wing assuming

isentropic flow.  You may take the gas constant for air as

R = 287 J/(kg K) and  = 1.4.								(8)

ME304   3/6

- Using the shock expansion method and the tables provided, calculate the lift

coefficient of the symmetric strut held at 4 incidence in supersonic flow

shown in Figure Q3.  Changes in the entropy through the oblique shock waves

may be neglected.											(20)

Figure Q3

- (a)		A ground to air guided missile accelerates from its launch ramp to

Mach 2.  It has unswept biconvex wings 8% thick held at a low

incidence which remain unchanged during flight.  Using clearly

labelled diagrams and graphs, discuss the major differences

occurring in the flow, pressure distribution and liftcoefficient over

the wings.										(16)

- How will the pitching moment on such an aerofoil be affected as the

missile accelerates?									(4)

ME304   4/6

- Before applying a CFD code to realistic flow calculations, a team of research engineers decided to test the code for a simple two dimensional potential flow.  The

x and y components of fluid velocity in this flow are vx = u = 2x and vy = v = -2y respectively.  You are asked to investigate analytically the properties of this

flow in order to compare them with the results of calculations.  Namely:

- Determine the stream function and plot the streamlines for  = 5 and

 = 10.											(5)

- If a uniform flow defined by  = 3y is superimposed on the above

flow, plot the resulting streamlines for  = 5 and  = 10.				(5)

- Determine the stream function and the velocity potential for the

above combined flow.									(7)

- The predictions of any CFD code need to satisfy the continuity

equation.  Write down the general form of this equation for a

stationary, incompressible flow.							(3)

ME304   5/6

- A flat plate is drawn submerged through still water at a velocity of 18 m s-1.

The plate is pulled so that its length is in the direction of motion and its width

is at right angles to the direction of motion.  If the plate is 6 m wide and 10 m

long, determine:

- The position of the laminar to turbulent transition.					(5)

- The total drag force acting on the plate.  (Assume that the density of

water  is 1000 kg m-3, its dynamic viscosity  is 10-3 kg m-1 s-1

and the transition takes place at Re = 105).						(10)

(c )		Derive the expression of the velocity in the viscous sublayer as a

function of the distance from the surface of the plate.  Start with the

general expression for a shear stress in the form					(5)

τo=μ∂u∂y

- The Prandtl mixing length theory has been an effective tool for the analysis of properties of the boundary layers in many engineering applications.

- What are the main assumptions of this theory?					(5)

- Derive the expression for flow velocity as a function of the distance

from the boundary using these assumptions and the relation

τturbulent=−ρvx'vy'

(assume that the flow is confined to the xy plane)					(10)

(c)		What is the main difference between k -  and the Reynolds stress 					turbulence models?										(5)

ME304   6/6

- Computational Fluid Dynamics (CFD) is widely accepted as one of the most effective tools in engineering design.  The answers to the following

questions are expected to demonstrate the depth of your understanding of the subject which is essential for practical applications.

- Using the finite difference approach derive the expression for the

second derivative

∂2u∂x2i,j

in a descretized form.									(10)

- Describe briefly the Gauss-Seidel point-by-point method.				(5)

(c)		Describe briefly the meaning of underrelaxation.					(5)