## Department of Electronics & Computer Engineering

# Mid-Session Test January 2005

## EE3052B - MultiMedia Signal Processing

## Time allowed 1 Hour Plus 10 minutes reading time

## Answer only three out of five questions

Ensure that your registration number is written clearly on the front cover.

1.

- Obtain the Fourier transform of a rectangular pulse with a duration of T seconds.

Sketch the magnitude of the Fourier transform of this pulse.

Find the main bandwidth of the spectrum of the rectangular pulse in terms of the pulse duration T.

Explain why the rectangular pulse and its frequency spectrum are so important in communication signal processing.

[6 Marks]

- State the minimum number of samples per cycle required to convert a continuous-time sine wave to a discrete-time sine wave.

Hence state the Nyquist-Shannon sampling theorem for sampling a continuous-time signal with a maximum frequency content of B Hz.

[2 Marks]

- Calculate the bit rate per second and the bandwidth required to transmit a digital music signal sampled at a rate of 44100 samples per second and with each sample represented with a 16-bit word.

[2 Marks]

## 2.

The discrete Fourier transform (DFT) equation is given by

X(k)=m=0N−1x(m)e−j2πNkm 	   k = 0, . . ., N1	Eq (1)

- Write the N complex sinusoidal basis functions of the DFT of    Equation (1)

[2 Marks]

- What is the physical interpretation of X(k) in terms of the vibrations of signal x(m).

Explain why X(k) needs to be a complex variable.

Express the discrete frequency variable k in terms of the actual frequency as a function of the sampling frequency Fs.

How would you interpret X(k) as the outcome of a correlation operation.

[4 Marks]

- Express the DFT transform equation in terms of a combination of a cosine transform and a sine transform.

[2 Marks]

- A music signal is sampled at a rate of 44100 samples per second. Calculate the DFT length needed to obtain a frequency resolution of 40 Hz. What is the resulting time resolution?

[2 Marks]

## 3.

- Briefly list the signal processing steps required in the window design technique, using the inverse Fourier transform, to obtain the coefficients of a causal finite impulse response discrete-time filter given the desired frequency response of the filter H(f).

[2 Marks]

- Using the inverse Fourier transform method design two digital filters, a low-pass filter and a high-pass filter, to divide the input signal into two equal bandwidth signals.

[5 Marks]

- With the aid of a sketch show how the low pass and high pass filters in (b) can be repeatedly used in a tree-like structure to divide a signal bandwidth into 3 sub band with the two lower bands having a width of half those of the upper band.

[3 Marks]

4.

- State three different applications of filters.

[1 Mark]

- Explain why the impulse response function can completely describe the characteristics of a linear time-invariant filter.

[2 Marks]

- Write the relationship between the impulse response and the frequency response of a linear time-invariant filter.

[2 Marks]

- A first second order digital filter is given by

y(m)= 1.386 y(m−1)−0.9604 y(m−2)+g x(m)

Obtain the z-transfer function of this filter and write the z transfer function of this equation in polar form in terms of the radius and angular frequencies of its zeros.

Sketch the pole-zero diagram and the frequency response of the filter.

[5 Marks]

5.

- Explain the relationships between the Laplace transform, the z-transform and the Fourier transform.

[2 Marks]

- Show how z-transform can be derived from Laplace transform and how discrete Fourier transform (DFT) can be derived from z-transform of a signal.

[3 Marks]

(c) The Figure below shows the discrete-time input signal and the impulse response of a linear time invariant filter. Using the principles of linearity and superposition, obtain the output of the filter in response to the input discrete-time input signal shown in the Figure.

[5 Marks]