7 Magnetic Field Sources

Take an element carrying current I and ask what are the properties of B at r with respect to the element.

r

I

dA

dB is perpendicular to dl, r. dB is in the right hand sense with respect to I, Iand 1r2

i.e.	dB=μ04πIdlxrr2=μ04πIdlxrr3

Where μ0=4πx10−7TmA	Magnetic permeability of vacuum

Straight Conductive Wire

z

a

dl = dz z                          ϕ        r

x

I

-a

r=xx−zzr=x2+z2

Hence dB=μ04πI.dzzxxx−zzx2+z232dB=μ0I4πdzx2+z232

Then as the field is only in the y direction

By=allElelmtsdBy=z=−az=aμ0I4πxdzx2+z232=μ0Ix4π−aa1x2+z232dz=μ0I4π2axx2+a2

As a becomes infinite B=μ0I2π1x

Force Between 2 Wires

I’

Segment of length l

r          B out

I

Bout=μ02πIr

Force on segment l of the top wire:	F=I'lxB=I'lBsin90 (dirn l x B)

F=I'lμ0I2πr

So like carrying wires are attracted.

Circular Loop

y

dl

a		The angle subtended by dl is ϕ

r=xx−ay

Z                                                   P

x

The element dl=dlzat the top of the loop.

dB=μ0I4πdlzx

By symmetry we can see that components of dB perpendicular to x-axis will cancel, as there are elements on the opposite side of the loop.

For our element:

dB=μ0I4πdlaxx2+a232

We need to some all around the loop, to give Bx

Bx=elementsdBX=elementsμ0I4πadlx2+a232Bx=μ0I4πax2+a232elementsdlBx=μ0I4πa2πax2+a232Bx=μ0Iπa2x2+a232

We can make Bx larger by increasing I or adding more loops (N)

Bx=μ0IN2a2x2+a232

Recalling μm=IA=NIπa2

Then	Bx=μ0μm2πx2+a232

Amperes Law

Recall B due to a straight wire B=μ0I2πr

I

B

B

So B.dl=Bdl=μ0I2πrdl=μ0I2πr2πr

So

B.dl=μ0Iat least for circles with I at there centre. What about when this isn’t the case:                               dl

B

B				 B, dl

r         dl

I

B        dl

B.dl=1B.dl+2B.dl+3B.dl+4B.dl

1B.dl=−μ0I2πr1dl=−μ0I2πr1dl=−μ0I2πr1α2πr1=−αμ0I3B.dl=μ0I2πr3dl=μ0I2πr3dl=μ0I2πr3α2πr3=αμ0I2B.dl=4B.dl=0

As B and dl are perpendicular.

So B.dl=−αμ0I+αμ0I+0+0=0

B.dl=	0	Arc – Radial Segments Outside I

μ0I	Path Encloses I

We will now generalise for any arbitrary path. By taking B.dl for a small segments.

B

B                                                                    dl

dθ                                                                        φ

dl

dθ

B.dl=Bdlcosφ	as dlcosφis projection onto circular arc

B.dl=μ0I2πrdθ

i.e. B.dl=μ0I2πdθB.dl=μ0I2π2πB.dl=μ0I

Likewise if we go outside current B.dl=0

Suppose we have I1, I2, I3…

B.dl=B1.dl+B2.dl+B3.dl+...B.dl=μ0I1+I2+I3+...

Amperes Law B.dl=μ0Ienclosed

Applications

e.g. Cylinder carrying uniformly distributed I

θ

I                                     z

“By symmetry” no θ or z dependence.

We know that B is perpendicular to I, so B=Bθθ

So chose circle as “Amperian Circle”

Amperian circle on the inside. Radius r < R

B.dl=Bθθ.rdθθB.dl=Bθr2π=μ0Ienclosed

Ienclosed=πr2πR2I

So Bθ2πr=μ0r2R2I	 Bθ

Bθ=μ02πrR2I

r > R	Bθ2πr=μ0IBθ=μ0I2πr	Just like a wire

R                       r

What if I is not uniform:

e.g	I=αr6z	r < R

Small area rdθdr

Amperian circle of r inside R.

We still have B=Bθθ

B.dl=Bθ2πr

Need to add up all the current through areas dA=rdθdr i.e. dA=rdθdrz

IEnclosed=circleαr6z.rdθdrzIEnclosed=α02πdθ0rr7dr=α2πr88

So Bθ2πr=α2πr88μ0⇒Bθ=μ0α8r7

For r > R

B.dl=μ0I.dlBθ2πr=μ0r=0Raα7dθdr+μ0Rr0rdθdrBθ2πr=μ02παR88Bθ=μ0α8R8r≡μ0Ienclose2πr

Solenoids

An infinite straight with n turns of wire per unit length, B does not depend on z or θ.

So B=Bzz

I

I

2

1

⊗	⊗

Take path 1

Top		B.dl=Bzl

Sides:		B.dl=0 As B is perpendicular to dl

Bottom: 	B.dl=−BZlAs zis antiparallel to dl

Therefore B.dl=00=μ0nlI−nlI0=0

Take path 2

B.dl=Boutl−BinlBoutl−Binl=μ0nlIBin=Bout−μonI

In fact Bout is independent of solenoid so field due to solenoid Bin=−μ0nIz

i.e. Bin=μ0nI

e.g.

Torodial Solenoid of n turns

B		By symmetry B is encircles the centre of each loop

Take path with circle of radius

⊗

⊗		⊗

⊗

B.dl=B2πrB2πr=μ0nIB=μ0nI2πr

Take limit of small windings and a large torus, we recover the formula for the infinite solenoid.

Magnetic Materials

B=B0+μ0M=B01+XM

B0 Externally applied

M Magnetism

Xm Susceptibility

B=kmB0	Where km is relative permeability

Replace μ0→μ0km=μ

Paramagnetic	km = 1x10-15

Diamagnetic	km = 1-10-5

Ferromagnetic	km ~ 105