Tutorial 4 (PDEs in the Real World)

Class work

- The temperature inside a square room, 5 metres x 5 metres, can be modelled using Poisson’s equation, ∇2u+g=0. Three of the walls (east, south & west) are heated with radiators running the length of the wall with the temperature fixed at 25˚C. The north wall is insulated. Determine the finite difference equations that can be used to solve for temperature:

  - In the centre of the room;

  - Along the north wall.

- The function u(x,y) satisfies the equation ∂2u∂x2+3∂2u∂y2+16=0 in the square domain bounded by 0 < x < 1 and 0 <  y < 1. The boundary values are given by:

∂u∂y=−u

Y

X

(1,0)

(0,0)

(0,1)

(1,1)

∂u∂x=0

u = 0

∂u∂y=0

Set up the discretised system representing the finite difference approximations using the grid spacing (x = y = 0.25). Write down the general finite difference formulas for the internal and boundary nodes and the specific formulas for each corner node.

Lab work

- Set up a finite difference model in Excel to the system in question 1 above using Gauss-Seidel, with x = y = 0.5. If g = 5, where is the hottest point of the room? Suppose g = –5; now where is the hottest point of the room?

- Solve the non-standard system in question 2 above using Gauss-Seidel in Excel. [N.B. This is the hardest question of the whole course.]

- Look at some of the websites listed in the lecture notes. Make a list of the application areas in which numerical methods have been used to solve PDEs.